Smart Appliance Communications Systems Requirements and Application-Level Evaluation Criteria

# Overview

This section describes the set of appliance application level selection criteria co-developed with input from AHAM.  The purpose of the application criteria is to identify major system level requirements for application level protocols that will help appliance manufacturers identify and differentiate relevant protocols for potential inclusion in future appliance designs.

The system level criteria were used to translate high-level major appliance customer expectations into specific application protocol capabilities.  For example, consumer expections such as “easy to use” and “no compromise of privacy” were used to derive a set of high level system requirements which included “proven interoperable protocol” and “security mechanisms”.  These system requirements were then used to define specific  application level capabilities such “open specification” and “supports session encyption” which were used to evlauate each protocol.

The application level system evaluation criteria used to evaluate appliance protocols are defined in the following sections.   Evalaution criteria are divided into major categories including “proven interoperable application”, “accepted technology in marketplace”, “acceptable cost trade-offs” and easy to provision to Home Area Network (HAN).  Each category and its criteria are described below.

# Contents

Overview	1

Contents	2

Proven Interoperable Application	4

IEC CIM compliance	4

Network Interface Exists	4

Loose coupling	4

Certification Bodies and Test Labs Exist	5

Supported and Vetted by Commercial Alliance	5

Offers an Open Specification	5

On the NIST Interoperability Roadmap	6

Supported and Vetted by SDO	6

Accepted Technology in the Marketplace	6

Current Installed Base	7

Technology currently in use in Non-HAN Systems	7

Technology currently in use in HAN Systems	7

Accepted Cost Trade-offs	8

Energy Savings versus Technology Cost	8

Years to Recover Costs	8

User configuration method exists without additional hardware	9

Pre-configurable	9

Easy to Provision to HAN/ Plug and Play Appliance	9

Supports Adding, Deleting, Modification of Devices	10

Supports Multiple Appliances	10

Supports Authentication	10

Minimize Enabling Infrastructure	10

Application Requires Additional Hardware	11

Application Requires Additional Software	11

Forward / Backward compatibility / Future proof	11

Supports authorized direct load control	11

Supports time-based power reduction/consumption	12

Supports power reduction to reduce energy costs	12

Supports third party load control	12

Supports load shifting/optimization	13

Supports independent power optimization	13

Interference and Noise Handling	14

Auto recovery from Loss of Communication	14

Supports Quality of Service (QOS) message prioritization	14

Supports Lossy/low power communications	14

Easy System Operation	15

Supports auto-discovery of devices	15

Auto-installation	15

Error Messages/Codes specified	15

Protocol specifies error handling	16

Easy to Maintain/Replace	16

Software Upgrades can be installed by service technician	16

Software Upgrades can be installed by consumer	16

Firmware Upgrades can be installed by service technician	17

Firmware Upgrades can be installed by consumer	17

Robust messaging capability	17

Supports 2 Way Communications	17

Supports General Data Queries	18

Supports Appliance-Specific Data Queries	18

Message verified with checksums/CRC algorithm/other	18

Supports non-static message formats	18

Intellectual property concerns / Licensing concerns	19

Licensing costs	19

Known litigation	19

Security Mechanisms	19

Supports session encryption	19

Supports user authorization	20

Survey Methodology	20

Communication Study Weights and Scoring	20

# Proven Interoperable Application

The Proven Interoperable Application criteria  helps the reader understand the overall interoperability potential of application level  protocols based on use of established standards, industry backing, and industry enforcement of interoperability.  Ideally, existing communications technologies selected should have demonstrated their ability to interoperate at key levels of the network stack.  Emerging protocols may have features designed to support interoperability.  Communications protocols should have a community of implementers and users as well as certified labs to demonstrate compliance.   The communications technology’s interoperability should ideally be established in the Smart Grid Community.

## IEC CIM compliance

This lower level Systems Requirement is to compare  application protocols on compliance with  International Electrotechnical Commission (IEC) Common Information Model (CIM) 61970 and 61968 standards.  IEC 61970 defines application level interfaces for energy management systems.   IEC 61968 defines information data exchanges between electrical distribution systems.   Application level protocols should leverage these two standards to increase the overall interoperability of appliance applications.

| High | Defined Protocol-to-CIM mapping specified |
| --- | --- |
| Medium | Some CIM mapping and/or stated intent to be CIM-compliant |
| Low | No mapping or intent to map to the CIM found in the protocol standard documentation set. |

## Network Interface Exists

This lower level Systems Requirement is to compare application protocols on their ability to  access a  network interface. Typically, an Internet Protocol (IP) interface is used to connect applications to a network. Although applications could potentially use a protocol other than IP for their network interface, but for the purposes of this evaluation IP is assumed to be the most interoperable network protocol.

| High | IP interface specified |
| --- | --- |
| Medium | Protocol is IP compatible |
| Low | No direct IP network interface support |

## Loose coupling

This lower level Systems Requirement is to compare  application protocols  on the issue of loose coupling between network layers. Loose coupling between network layers means that the layers behavior is independent of other layers.  Loosely coupled network layers interact only through well defined interfaces. Loosely coupled interfaces do not make assumptions about the other network protocol layers.  In particular, loosely coupled network layers do not assume that a specific related protocol is used for the other network layers.

| High | Protocol assumes no coupling of layers. Protocol does not assume use of any related network layers. |
| --- | --- |
| Medium | Protocol allows coupled network layers. Protocol allows use of related non-independent layers. |
| Low | Protocol specified coupled network layers. Protocol assumes other network layers use specific related technology. |

## Certification Bodies and Test Labs Exist

This lower level Systems Requirement is to compare  applications protocols  on the  available bodies to certify correct interoperable implementation of the protocol.  Certification shall be enforced through execution of documented repeatable test procedures.  Multiple test and certification labs shall exist for a specific technology. Certification and test specifications shall be open to those considering or seeking certification under reasonable terms.

| High | Established independent certification process |
| --- | --- |
| Medium | Certification method requirements are documented |
| Low | No known certification method |

## Supported and Vetted by Commercial Alliance

This lower level Systems Requirement is to compare  application protocols on supported and vetted by a commercial alliance whose purpose is to contribute, govern, advocate, and enforce adoption of features into a given technology specification in the commercial market.  Historically, SDOs do not market, certify, or directly develop technology, but govern and record the standard and its establishment in the most open and fair way.  Commercial alliances represent a healthy users group (typically vendor led) for those seeking to add features and robustness to a technology, and are often used for certification oversight.

| High | Free and open membership alliance group. |
| --- | --- |
| Medium | Fee-based alliance group exists. |
| Low | No identified alliance/user group. |

## Offers an Open Specification

This lower level Systems Requirement is to compare  protocols on the issues of open specification.  Each individual communications protocol shall have an established specification.  Appliance manufacturers must be able to obtain copies of the specification at a reasonable cost.  The specification shall be governed by an SDO or other commercial organization of which the manufacturer may choose to join.  The specification shall have a process by which it may be extended, modified, or revised.

| High | Free and open standard |
| --- | --- |
| Medium | Standard is not proprietary but obtaining a copy requires membership or a fee |
| Low | Standard is proprietary |

## On the NIST Interoperability Roadmap

This lower level Systems Requirement is to compare  application protocols on the issue  of announced as accepted or considered on the NIST Interoperability Roadmap.  A significant panel of industry companies and experts were leveraged to establish and govern acceptance into the list.   As the list was defined to catalyze Smart Grid technology uptake, its constituents are obviously relevant to Smart Grid use and should be considered with priority by Appliance Manufacturers.

| High | Technology has been approved by NIST/SGIP as interoperable for use in the Smart Grid. |
| --- | --- |
| Medium | Technology is being considered by NIST/SGIP for inclusion in the interoperable Smart Grid technology list. |
| Low | Unknown OR technology is not being actively considered by NIST/SGIP as an interoperable Smart Grid technology. |

## Supported and Vetted by SDO

This lower level Systems Requirement is to compare application protocols on the issue of being supported and vetted by an accredited standards definition organization (SDO).  Standards development organizations represent national interest in the long term interoperability of the underlying technology.  Use of an accredited SDO nearly establishes the required governance, openness, and community of relevance for a technology.

| High | Accredited SDO has approved the standard |
| --- | --- |
| Medium | Accredited SDO identified, but standard not yet accepted/approved |
| Low | SDO not identified |

# Accepted Technology in the Marketplace

The Accepted Technology in the Marketplace criteria helps the reader understand how well established the application protocols are in the existing marketplace.  Appliance applications should be relevant to the markets in which they seek to participate. Ideally protocols used in appliance applications should have a current installed user base of products.

## Current Installed Base

This lower level Systems Requirement is to compare  application protocols on the  use in existing products.  Ideally protocols used in appliance applications would be used in existing appliances already on the market. However, as smart grid-enabled appliances are not readily available in the marketplace; the use of new protocols specifically addressing smart grid capabilities is still in the product evaluation stage. This criterion evaluates the use of the protocol in any application and specifically allows use of the protocol in devices other than appliances.

The use of existing technology has the potential to effectively leverage proven protocols. Proven protocols provide potential benefits to consumers especially in terms of reliability and feature set.  Ideally protocols will be current use in the home environment. Protocols were evaluated in terms of current use in both HAN and non-HAN systems.

| High | More than 100 known installations using the protocol. An installation is defined as an application at a specific location. |
| --- | --- |
| Medium | Between 11 to 100 installations using the protocol. An installation is defined as an application at a specific location. |
| Low | 10 or fewer installations using the protocol. An installation is defined as an application at a specific location. |

## Technology currently in use in Non-HAN Systems

This lower level Systems Requirement is to compare  applications on  leverageing existing application software where possible. Existing applications can be effectively used to implement appliance specific systems.  This criterion evaluates the use of the protocol in existing non-HAN systems.

| High | Technology currently in use in products designed for home environments. |
| --- | --- |
| Medium | Technology currently in use in commercial environments products, but not in products designed for home environments. |
| Low | Technology not currently in known use in products designed for either the home or commercial environments. |

## Technology currently in use in HAN Systems

This lower level Systems Requirement is to compare  applications leverage existing HAN application software where possible. Existing  HAN applications can, and should be, used to implement appliance specific systems for the home.  This criteria evaluates the use of the protocol in existing HAN systems.

| High | Technology currently in Home Area Network (HAN) products. |
| --- | --- |
| Medium | Technology currently in HAN prototypes available for evaluation. |
| Low | Technology not currently available for evaluation and/or protocol specification version 1.0 not yet approved. |

# {  You left out explanations of the general data queries and the appliance specific queries   }

Moved to Robust Message Capabilitity. Please see explanation in that section.

Accepted Cost Trade-offsThe Accepted Cost Trade-offs criterion evaluates factors which might, or might not, impact the eventual cost to the consumer of a smart appliance.  The reader is cautioned that consumer cost factors will vary greatly due to differences in the cost of electricity which varies widely, and by special incentives which may or may not be offered to consumers.   Note further that as AHAM is a trade organization representing the majority of appliance manufactures in the United States, it is bound by anti-trust laws which impact the ability to evaluate costs.

Cost factors considered in this category are the consumer’s, the ability to pre-configure devices, the ability to configure devices without the need for additional consumer hardware, and the complexity of the standard.

## User configuration method exists without additional hardware

This lower level Systems Requirement is to compare  applications ability to  be configurable without requiring additional hardware. Consumers should not need additional hardware in order to configure a new appliance for Smart Grid use. The need for additional hardware will tend to increase consumer cost.  { what assumptions are baked into this?  Just to install a new appliance assuming applications protocol XYZ is installed in the appliance, assumes what about the envionment ….?  }

| High | No known additional hardware required for configuration |
| --- | --- |
| Medium | Additional external hardware needed for configuration such as a server |
| Low | Additional in-home equipment, such as an adapter, needed to configure devices |

## Pre-configurable

This lower level Systems Requirement is to compare applications ability to be pre-configurable. Pre-configurable devices can be configured at the factory or other location with appropriate parameters so that consumers will have few required configuration steps.  The ability to pre-configure devices should tend to reduce consumer costs.   {what needs to be pre-configured?  (e.g. certification / security information?)  }

|  | High | Devices can be pre/auto configured. |  |
| --- | --- | --- | --- |
|  | Medium | Some automation of configuration e.g. configuration parameters reusable/extendable. |  |
|  | Low | Manual configuration required. |  |
| ## Standard Implementation Complexity |  |  |  |

This lower criteria evaluates the complexity of implementing the protocol as specified in its SDO-based documentation.  The SDO-based documentation was reviewed to determine how difficult engineering and implementing the protocol would be in new applications.  Standard complexity was assessed based on the readability, use of common known techniques and length of the standard.

| High | Standard documentation easy to comprehension which uses existing common known techniques |
| --- | --- |
| Medium | Standard documentation and technology usage were mixed e.g. complex specification and low implementation of common techniques or less complex specification which uses new technology/methods. |
| Low | Standard documentation is complex and technology/methods use or less well known techniques. |

# Easy to Provision to HAN/ Plug and Play Appliance

The Easy to Provision to HAN/Plug and Play Appliance category evaluates factors which would allow consumers to easily add a smart appliance to their home. Factors considered include the ability to add new appliances and the ability to integrate a new appliance which includes provisioning, a term which refers to integrating the device into the consumers Home Area Network (HAN).   Appliance applications should ideally support plug and play capabilities which would require a minimum of consumer effort to install and configure new appliances

## Supports Adding, Deleting, Modification of Devices

This lower level Systems Requirement is to compare  applications that allow consumers to add new appliances. Appliance applications shall allow consumers to remove appliances from their HAN.  Appliance applications shall allow consumers to remove appliances from participation in programs. Appliance applications shall allow consumers to modify settings for existing appliances.

| High | Protocol supports multiple devices of multiple distinct types. |
| --- | --- |
| Medium | Protocol supports multiple devices. |
| Low | Protocol provides no known support for individual devices. |

## Supports Multiple Appliances

This lower level Systems Requirement is to compare applications ability to support multiple devices. In the future, HANs are expected to potentially grow to include many devices. Thus the need to be able to support a large number of devices is relevant for appliance applications, and number of supported devices is evaluated here.

| High | Protocol can support 100 or more devices. |
| --- | --- |
| Medium | Protocol can support 11 to 99 devices. |
| Low | Protocol can support 10 devices or fewer devices |

# Minimize Enabling Infrastructure

This lower level Systems Requirement is to compare  applications ability to allow consumers to acquire the new technology with a minimum of additional extra components.  Ideally users will be able to configure their smart appliance with no additional hardware or software.   Consumers are familiar with configuring devices such as programmable thermostats which do not require additional hardware or software. Ideally this implementation model can be used with appliances.

## Application Requires Additional Hardware

This lower level Systems Requirement is to compare protocol’s ability to be configurable without additional external hardware. Ideally configuration of appliances can be accomplished by consumers directly interacting with the appliance e.g. though simple controls contained on the appliance itself.

| High | Appliances can be configured with no known additional external hardware. |
| --- | --- |
| Medium | Appliance configuration requires additional hardware such as a server, but consumers do not need such equipment in their homes. |
| Low | Appliance configuration requires additional in-home equipment such as a PC. |

## Application Requires Additional Software

This lower level Systems Requirement is to compare protocols ability to be configurable without additional software. Ideally configuration of appliances can be accomplished by consumers directly interacting with the application though simple controls such as setting an application parameter e.g. energy conservation level.

| High | Appliances can be configured with no known additional software |
| --- | --- |
| Medium | Appliance configuration requires additional software such as a web interface, but consumers do not need a copy of the software. |
| Low | Appliance configuration requires additional in-home software such as a browser supporting an HTTP interface. |

# Forward / Backward compatibility / Future proof

Appliance applications should use protocols in which are compatible with existing and future technologies. Backward-compatible appliances can interact with existing technology while forward compatible appliance will be able to interact with technology over the lifetime of the appliance.  For appliance applications, these criteria measured the ability to support existing and planned price methods, rates and programs.

## Supports authorized direct load control

Appliance applications shall support direct load control programs where possible. Direct load control programs allow an authorized organization to decrease or limit power usage during certain prescribed conditions. Consumers typically sign up for direct load control programs in exchange for monetary compensation. For example, in exchange for an annual payment consumers allow their local utility to decrease power consumption for particular house hold devices during peak load periods.

| High | Specific support for direct load control is provided. |
| --- | --- |
| Medium | Limited support for direct load control is provided, which would allow implementation of some load direct load control programs. |
| Low | No support for direct load control provided. |

## Supports time-based power reduction/consumption

Appliance applications shall support time-of-use rate programs where possible. Time-based rates vary the cost of electricity to consumers based on the time of day according to a fixed published schedule.  Time based power reduction allows consumers to reduce their power costs by scheduling usage during low cost periods. For example, refrigerator defrost cycles would be set to run during the least expensive time period.

| High | Specific support for time-of-use rates is provided. |
| --- | --- |
| Medium | Limited support for time-of-use is provided, which would allow implementation of some load time-of-use rates. |
| Low | No support for time-of-use rates provided. |

## Supports power reduction to reduce energy costs

Appliance applications shall support power reduction to reduce energy costs where possible.  Some areas vary energy costs to the consumer based on real-time pricing.  Real-time pricing more accurately reflects the true cost of producing energy in the cost paid by consumers.  For example, an energy provider can transmit energy cost data hourly. Based on costs, consumers could choose to use appliances or not.

| High | Specific support for real-time rates is provided. |
| --- | --- |
| Medium | Limited support for real-time rates is provided, which would allow implementation of some real-time pricing. |
| Low | No support for real-time rates provided. |

## Supports third party load control

Appliance applications shall support third party load control programs where possible. In addition to local utilities and other power providers with load control programs, some areas also allow so-called third parties to implement direct load control programs. Typically these programs use direct load control for a combined or “aggregated” set of users to ensure load reduction during peak loads. These third-party companies are frequently referred to as “aggregators”.  For example a third party company may enroll a large number of consumers who agree to decrease energy usage during certain prescribed times in exchange for a monetary payment.

| High; Specific support for third party load control is provided.. Medium; Limited support for third party load is provided, which would allow implementation of some load direct load control programs.. Low; No support for third party load provided.. |  |
| --- | --- |

## Supports load shifting/optimization

Appliance applications shall support load shifting for load optimization where possible. Load shifting delays power consumption to more optimal load periods.  For example, battery charge cycles could be either delayed during high usage periods or charge cycles could be increased during low usage periods.

| High | Specific support for load shifting is provided. |
| --- | --- |
| Medium | Limited support for load shifting is provided, which would allow implementation of some load shifting. |
| Low | No support for load shifting is provided. |

## Supports independent power optimization

Appliance applications shall support independent power optimization where possible. In addition to formal load optimization programs, the need for independent consumer-based power optimization exists. For example, consumers with solar panels may want to maximize use of power when the sun is shining even during a time when system load is high because this best optimizes the use of renewable natural resources.

| High | Specific support for independent power optimization is provided. |
| --- | --- |
| Medium | Limited support for independent power optimization is provided, which would allow implementation of some independent power optimization. |
| Low | No support for independent power optimization is provided. |

# Interference and Noise Handling

Appliance applications should use protocols with support for operation in typical real-world environments.  Applications will typically encounter loss-of-communications between devices, as well as the need to send some application-level messages with higher priority.  These criteria measure the ability of the application layer to operate in typical home environments.

## Auto recovery from Loss of Communication

This lower level Systems Requirement is to compare applications ability to recover from loss-of-communications. Communication failures of individual messages can lead to a loss of communication between devices. For example, an appliance may not be able to communicate through the HAN to the consumer.

| High | Specific LOC recovery methods are specified in the specification’s documentation. |
| --- | --- |
| Medium | LOC handling is included/required by the specification. |
| Low | No support for LOC processing is provided in the specification. |

## Supports Quality of Service (QOS) message prioritization

This lower level Systems Requirement is to compare to allow higher priority messages to be transmitted ahead of lower priority messages. Appliance applications shall provide Quality of Service (QOS) message prioritization.  Application messages are typically not of equal importance. For example, a routine heartbeat message is typically not as critical as a safety-related message.

| High | Specific QOS message prioritization is provided in the specification’s documentation. |
| --- | --- |
| Medium | Limited QOS message prioritization is provided in the specification’s documentation. |
| Low | No support for QOS processing is included in the specification. |

## Supports Lossy/low power communications

This lower level Systems Requirement is to compare applications ability to  provide support for lossy/low power networks.  Appliance networks are expected to be characterized as having lossy/low power properties.  {is this a power  limiting feature?  Need clearer definition of what this is }  Lossy/low power networks typically are composed of multiple devices with limited resources.  Such networks, including HAN networks with appliances, should be designed to work well given limited power, bandwidth and/or processing power.

| High | Specific lossy/low power logic is provided by the protocol. |
| --- | --- |
| Medium | Limited lossy/low power logic is provided by the protocol. |
| Low | No support for lossy/low power networks is included in the protocol. |

# Easy System Operation

Easy system operation allows consumers to install and use appliance applications effectively. Features which enable easy use by consumers includes automatic discovery of new appliances, automatic installation and error messages in human-readable format.

## Supports auto-discovery of devices

This lower level Systems Requirement is to compare applications ability to provide automatic discovery of new appliances.  Automatic discovery of new appliances will help consumers add new devices into their HANs.

| High | Devices auto-discovered. |
| --- | --- |
| Medium | Devices need not be individually specified. |
| Low | Devices must be individually specified. |

## Auto-installation

This lower level Systems Requirement is to compare  applications ability to provide support for automatic installation of appliance configuration.  Automatic installation of appliance configuration parameters will help consumers add new devices into their HANs.

| High | Protocol features include automatic installation of configuration data. |
| --- | --- |
| Medium | Devices need not be individually configured. |
| Low | Manual installation of configuration required. |

## Error Messages/Codes specified

This lower level Systems Requirement is to compare applications ability to provide human-readable error codes to help consumers add, delete and modify devices in their HANs.   Human readable error codes help consumers correct problems with or without the use of other technical support.

| High | Specific error codes/messages defined in the protocol specification documentation. |
| --- | --- |
| Medium | Protocol requires error codes, specific error codes not defined in the protocol specification documentation. |
| Low | Error codes not defined/specified in the protocol specification documentation. |

## Protocol specifies error handling

This lower level Systems Requirement is to compare  applications ability to provide error handling.  Error handling helps consumers correct problems with or without the use of other technical support.  Lack of error handling information in the specification can lead to varying implementations, rather than a consistent, standards-based error handling approach.

| High | Specific error handling defined in the protocol specification documentation. |
| --- | --- |
| Medium | Protocol mentions error handling, specific error handling not defined in the protocol specification documentation. |
| Low | Error handling not defined/specified in the protocol specification documentation. |

# Easy to Maintain/Replace

Appliance application should be upgradable by consumers. This set of criteria measures the ability of consumers and field technicians to upgrade application software and firmware.

## Software Upgrades can be installed by service technician

This lower level Systems Requirement is to compare applications ability to be upgradable. This criterion measures the ability of service technicians to upgrade software using specialized equipment if necessary.

| High | Specific software upgrade methodology defined in the protocol specification documentation. |
| --- | --- |
| Medium | Software upgradable, no specific upgrade method specified. |
| Low | No method to upgrade software mentioned in the protocol specification documentation. |

## Software Upgrades can be installed by consumer

This lower level Systems Requirement is to compare applications ability to be upgraded preferably by consumers without specialized equipment. This criterion measures the ability of consumers to upgrade software without use of specialized equipment.

| High | Consumer can upgrade software in less than 10 minutes. |
| --- | --- |
| Medium | Software upgrade possible without specialized equipment. |
| Low | No method to upgrade software without specialized equipment. |

## Firmware Upgrades can be installed by service technician

This lower level Systems Requirement is to compare  application  firmware ability to  be upgraded. This criterion measures the ability of service technicians to upgrade firmware using specialized equipment if necessary.

| High | Specific firmware upgrade methodology defined in the protocol specification documentation. Firmware upgradable remotely. |
| --- | --- |
| Medium | Firmware upgradable, no specific upgrade method specified. |
| Low | No method to upgrade firmware mentioned in the protocol specification documentation. |

## Firmware Upgrades can be installed by consumer

This lower level Systems Requirement is to compare  application firmware ability to be upgraded. preferably by consumers without specialized equipment. This criterion measures the ability of consumers to upgrade firmware without use of specialized equipment.

| High | Consumer can upgrade firmware remotely in less than 10 minutes. |
| --- | --- |
| Medium | Firmware upgrade possible without specialized equipment. |
| Low | No method to upgrade firmware without specialized equipment. |

# Robust messaging capability

Robust messaging capability ensures that data messages are verified during transmission, that messages in non-static formats are supported by the protocol, and that message data queries including appliance-specific queries are supported. Non-static messages allow for more new messages to be used which are not necessarily similar to current messages. Typically non-static message are defined using Extensible Markup Language (XML) or similar methodology.

## Supports 2 Way

This lower level Systems Requirement is to compare application protocols ability to support two way communications. Two way communications allow the appliance to return at least limited status in response to messages, e.g. a status indicating that a command has been received.

| High | Protocol supports two-way communications. |
| --- | --- |
| Medium | (not used) |
| Low | Protocol does not support two-way communications. |

## Supports General Data Queries

This lower level Systems Requirement is to compare application protocols ability to support data queries. Data queries allow the appliance to return at status in response requests for specific data items, e.g. amount of energy consumed over a time period.

| High | Data queries supported including queries on energy usage |
| --- | --- |
| Medium | General data queries supported , no known queries for energy usage |
| Low | Protocol does not support data queries for specific data items on request |

## Supports Appliance-Specific Data

This lower level Systems Requirement is to compare application protocols ability to support appliance-specific data queries. Appliance-specific data queries allow the appliance to return responses to requests for appliance-specific data items, e.g. freezer temperature.

| High | Appliance specific data queries supported. |
| --- | --- |
| Medium | Limited support which could be used for appliance-specific requests such as some support for device specific queries. |
| Low | Protocol does not support data queries for device-specific data items |

## Message verified with checksums/CRC algorithm/other

This lower level Systems Requirement is to compare application ability to verify message content.  Various message verification methods include using checksums, cyclic redundancy check (CRC) or other verification method to help ensure that data transmission errors are detected.

| High | Method to both detect and correct message errors specified in the protocol specification documentation |
| --- | --- |
| Medium | Single message failures detectable. |
| Low | No message verification methodology mentioned in the protocol specification documentation. |

## Supports non-static message formats

This lower level Systems Requirement is to compare applications ability to allow definition of non-static message formats.  Non-static messages allow new messages which are not necessarily similar to current messages, and also facilitates interfaces with other systems.

| High | New non-static messages can be added in future. |
| --- | --- |
| Medium | Messages defined, new static messages possible. |
| Low | Messages defined with no ability to add new messages. |

# Security Mechanisms

Appliance application should use appropriate security measures to ensure that consumers have control of their appliances, and that unauthorized persons do not. Encryption is used to ensure that consumer information is not visible to unauthorized persons. Authorization logic is used to grant privileges to consumer using access controls. Authentication is used to verify that users are who they represent themselves to be. Authentication logic is typically implemented with passwords.

## Supports session encryption

This lower level Systems Requirement is to compare applications ability to use encryption.

| High | Encryption support specified in protocol. |
| --- | --- |
| Medium | Encryption could be used. |
| Low | No encryption methodology mentioned in the protocol specification documentation. |

## Supports user authorization

This lower level Systems Requirement is to compare  application ability to provide authorization privileges. Authorization specifies access rights to functionality. For example, in a household only some users may be allowed to change configuration data.

| High | Access control/authorization required. |
| --- | --- |
| Medium | Access control/authorization could be used. |
| Low | No authorization method specified. |

## Supports Authentication   {isn’t this a security issue?  Maybe Move under Security Mechanisms?

This lower level Systems Requirement is to compare applications ability to support a mechanism to ensure only authorized people can configure and provision devices.  Consumers will not accept appliances that are not under their control; hence authentication of anyone trying to add, delete or modify appliance settings is required.

| High | Protocol supports multiple methods to authenticate people, including methods in additional to passwords. |
| --- | --- |
| Medium | Protocol supports passwords or other method to authenticate people. |
| Low | Protocol documentation specified no known authentication method. |

# Survey Methodology

## Communication Study Weights and Scoring

The AHAM communication study evaluated existing standards for both the physical and application layers of the Home Area Network (HAN) to Smart Appliance interface.  In commonly used network models, the physical layer performs the actual message transmission, while the application layer performs higher level tasks such as controlling energy use in real time.

Several of the standards evaluated, including ZigBee, contain both application and physical layer requirements. The communications study methodology separately evaluated the suitability of the physical and application level functionality for all protocols including those standards which covered both.

In order for Smart Appliances to be used effectively in the Smart Grid, consumers must be willing to purchase and use them.  As appliances are consumer products, this communications study evaluated existing standards in terms of consumer wants and needs. Existing technologies from the NIST standards list and other applicable technologies as identified by EnerNex were evaluated in terms of how well they met consumer needs for Smart Appliances.  We used the Quality Function Deployment (QFD) as the evaluation methodology. As stated in the The House of Quality4, the intent of QFD is to design, manufacture, and market products that “customers want to purchase and will continue to purchase”.

Using QFD methology, consumer expections such as “easy to use” and “no compromise of privacy” were used to derive a set of high level system requirements which included “plug and play, “proven protocol” and “forward compatibility”.  These consumer expectations were provided by the AHAM members based on their experience with appliacne consumers and markets.   In Table 1, consumer expections are shown in the rows and the system requirements are shown in the columns.  The relative importance from the consumers viewpoint was assigned a value of 1 to 5 with 5 being high.  A value of 5 (high) was assigned to the consumer expections for safety, privacy and security.  The remaining consumer expectations were assigned a value of 3 (medium).  These relative importance factors were again provided by the AHAM members based on experience with appliance consumers and markets.

Next, the AHAM team developed the list of appliance system level operatioal requirements related to protocols that would influence the consumer expectaions.   Then each consumer expection/system requirement pair was assigned a value indicating the importance of the requirement to the consumer expection.    For example, the “Easy to Install/Plug and Play” pair was assigned a value of high ( which was assign a wighit of 9) because  the Plug and Play requirement is highly corrolated with the consumer expection of Easy to Install.  The total of the importance score for each system requirement was calculated.  Total scores less than 100 were assigned a Low value (weight of 1), scores between 100 and 200 were assigned a Medium value (weight of 3) and scores greater than 200 were assigned a value of High (weight of 9).

The EnerNEx team then developed lower level requirements derived from the high level system requirements for both the physical and application layers.  For example “proven protocol”  was decomposed into requirements including “Standards Development Organization (SDO) supported” and “Certification Bodies / Test Labs Exist”.  Table 2 shows the derived requirements and the evaluation criteria for each physical requirement and Table 3 shows the same data for the application requirements.  Each protocol was evaluated to determine if it met the requirement.  Values of High, Medium and Low were assigned based on a defined set of criteria.  Table 4 shows the Physical layer requirements, evaluation criteria and results and Table 5 shows the same information for the Application level.

The QFD methodology was  used to assign a numerical score for each protocol.   A normalized score was used which assigned the same total weight to each high level system requirement regardless of the number of derived lower level requirements.  The numerical values for Low, Medium and High are 1, 3, and 9 respectivally.

The formula for each numerical score is as follows:

CustomerImportanceScore * NormalizationFactor * Evaluation Score

where Low = 1, Medium = 2 and High = 9.

For example, the Proven Protocol/IEC Common Information Model (CIM) was assigned an importance value of 3 (Medium) in Table 2, and a normalization factor of 0.14 because there are 7 sub items in the major category (1/7). If a protocol earned a High in the assessment, it’s score would thus be :

CustomerImportanceScore * NormalizationFactor * Evaluation Score

3  * .14 * 9 = 11.3

Each protocol was evaluated for each derived requirement, and the results totaled.  The total score for each system requirement (row) is also in Tables 4 and 5 to give the reader an indication of the importance of that requirement to the overall set of requirements, with the higher the score the more important that requirement was to the overall score.

Table 1. Customer Expectations and System Requirements

(QFD1 tab data)

Table 2. Physical Layer Requirements

(QF2 tab data)

Table 3. Application Layer Requirements

(QFD2b App Definitions tab data)

Table 4. Physical Layer Evaluation Results

(QFD2a Sys to Tech tab data)

Table 5. Application Layer Evaluation Results

(QFD2b Sys to App Protocol tab data)