Smart Appliance Communications Systems Technology Evaluation Criteria

# Overview

This document describes a set of appliance communications PHY/LINK/NET systems level requirements and  technology selection criteria co-developed with between EnerNex and the Association for Home Appliance Manufacturers (AHAM).  AHAM focused on consumer driven higher level systems requirements while EnerNex focused on detailing out the system level requirements and evaluation of the protocol to those requirements.  The purpose of the evaluation criteria is to provide consumer driven differentiators in PHY/LINK/NET communications technologies that help appliance manufacturers identify relevant technologies in the space for potential inclusion in future appliance designs.  These system level criteria have been subsequently used to translate high-level major appliance customer expectations into specific communications technology offerings.  No two technologies have the same layer functional offering, network stack coverage, or design purpose.Representing them into a lateral comparison consistent with appliance manufacturer vision was non-trivial.

Communications technologies are easily described and compared in a more quantitative space such as frequencies, modulation means, channels and other dependent factors, but such descriptions are harder to map in an understandable high-level fashion into the space of appliance manufacturer and customer requirements.  Even if technologies were fully described in a technical capability space,  translation of this information into meeting consumer expectations is not always straight forward.  To this point, the appliance manufacturer’s system level requirements for communications technologies will not change, while the amount of technical variation within vendors of a single technology or set of technologies can be high.   We have attempted to make criteria and assessment boundaries that can be used to represent technology offering as a whole rather than by unique vendor.

To help those seeking to apply the criteria and their sub categories to technologies on behalf of appliance manufacutures, we provide a subjective ranking mechanism of high, medium, and low with descriptions of AHAM’s consensus on the meanings of the subjective rankings in use for technology evaluation.

Contents

Overview	1

1) Proven Interoperable PHY/LINK/NET	5

1A) Reuses the Standards Basis	5

1B) Offers an Open Specification	5

1C) Is Proven and Testable	6

1D) Has Certification Bodies and Test Labs	6

1E) On the NIST Interoperability Roadmap	6

1F) Supported and Vetted by SDO	7

1G) Supported and Vetted by Commercial Alliance	7

1H) Capable of Delivering Smart Energy Services & Protocols	7

4) Easy to Provision / Plug-and-Play Appliances	8

4A) Endpoints Easy to Join to a Network	8

4B) Secure Joining to Network Easy	8

4C) Integrated Support for PHY Extenders / Repeater Support	9

4D) Integrated Support for Change of PHY / Bridging Support	9

4E) Supports Provisioning User Feedback	9

13) Affordable to Install	10

5) Minimizes Enabling Infrastructure	10

5A) Enables Low Power Consumption	10

5B) High Single Station Coverage	11

5C) Requires Minimal Additional Cabling	11

2) Accepted Technology in the Marketplace	12

2A) AMI / Electric Meter Vendor Use	12

2B) Home Internet Gateway Use	12

2C) DR Gateway Use	12

2D) Protocol Used by Consumer Electronics	12

2E) Other HAN Installed Base Vendor Use	13

3) Acceptable Cost Trade-offs	13

3A) Low Cost Per Physical Station	13

3B) Sufficient Data Bandwidth Available	14

3C) Sufficient Data Latency Available	14

3D) Energy Proportional Power Consumption	14

15) Sufficient Enabling Security Mechanisms	15

15A) Link Encrypted	15

15B) Network Encrypted	15

15C) Endpoints Mutually Authenticatable	16

15D) Supports PKI	16

15E) Supports PSK	16

15F) Supports Logical Separation	16

6) Technology Supply Chain Exists	17

6A) Integration Talent Exists	17

6B) Integration Tools Exist	17

6C) Part Availability	17

6Ca) Multiple Silicon Implementers Identified	17

6Cb) Silicon Availability	18

6Cc) Software Availability	18

7) Future Proof	18

7A) Backward Compatibility	18

7B) Firmware Upgrade Exists	18

8) Handles Interference and Noise	19

8A) Scales to Multiple Dwelling Unit Environments	19

8B) Coexistence Mechanism Used	19

8C) Supports Transmit Power Control	20

8D) Supports Spectrum Spreading	20

9) Easy System Operation	20

9A) Enables Stable and Reliable Links	20

9B) Provides Link State Feedback	21

12) Robust Messaging Capability	21

12A) Supports and Enforces Quality of Service (QoS) Delivery	21

12B) Offers Error Correcting Codes	21

10) Easy to Maintain	22

11) Easy to Replace	22

1) Proven Interoperable PHY/LINK/NET

This criteria helps the reader understand the overall interoperability potential of the technology based on long term standards basis, industry backing, and industry enforcement of interoperability for the technology under discussion.  Existing communications technologies selected should have well demonstrated their ability to interoperate at key levels of the network stack.  Emerging technologies may have features that are being designed to support interoperability.  Communications technologies should  have a community of implementers and users to establish and enforce interoperability.   The communications technology’s interoperability should be established in the Smart Grid Community.

## 1A) Reuses the Standards Basis

This lower level Systems Requirement is to compare  communications technologies that  leverage the standards basis.  Communications technologies often make the mistake of unneccessarily reinventing the wheel when developing their functional components. Technologies that leverage the existing standards basis are more easily understood, more likely to interoperate, and offer solid foundation on which to build or extend a communications architecture.

| High | A relatively high degree of standards basis reuse exists for the technology. |
| --- | --- |
| Medium | An average degree of standards basis reuse exists for the technology. |
| Low | Unknown OR A low degree of standards basis reuse exists for the technology. |

## 1B) Offers an Open Specification

This lower level Systems Requirement is to compare  communications technologies on the existence of  open specification.  The individual communications technology shall have an established specification.  Appliance manufacturers shall be able to obtain a copy of the specification.  The specification shall be governed by  an SDO or other commercial organization of which the manufacturer may choose to join.  The specification shall have a process by which it may be extended, modified, or revised.  The specification may be implemented by under reasonable and non-discriminatory licensing (RAND) terms by all interested manufacturers.  Implemnetation of the specification shall indicate some degree or expectation of interoperability with other products implementing the specification.

| High | The technology has a specification. The specification was created and is controlled in an open and fair process by an association or a standardization body intending to achieve interoperability and interchangeability in industry. An open specification is not controlled by a single company or individual or by a group with discriminatory membership criteria and is open to new membership . Copies of Open Specifications are available free of charge or for a moderate fee and can be implemented under reasonable and non discriminatory licensing (RAND) terms by all interested parties. It is likely that implementing the specification creates a degree of interoperability. |
| --- | --- |
| Medium | The technology has a specification. The specification was created and controlled in an open and fair process, by an association or a standardization body intending to achieve interoperability and interchangeability. There may be issues with terms of implementation or the foundation and control of the specifying body. It is likely that implementing the specification creates a degree of interoperability. |
| Low | Unknown OR The technology might not have a clear interoperable specification. There might not be a specification body. There are issues in the separation of individual vendor and specification body. The specification body may not have open documented processes of technology revision process. Terms of implementation may not be established or may be unfair. |

## 1C) Is Proven and Testable

This lower level Systems Requirement is to compare communications technologies on there ability to be  evaluated by the manufacturer prior to selection.   Technologies under development are often not yet available to evaluate and test.  Manufacturers are seeking to evaluate technology prior to acquisition to assure customer value.

| High | Instances of the technology are easy to obtain and test. |
| --- | --- |
| Medium | Instances of the technology are more difficult to obtain, must be obtained direct from vendors, and may be difficult to test. |
| Low | Uknown OR Few instances of the technology may be found or are currently testable. |

## 1D) Has Certification Bodies and Test Labs

This lower level Systems Requirement is to compare communications technologies  that have available bodies to certify correct interoperable implementation of the communications capability.  Certification shall be enforced through execution of documented repeatable verifiable test procedures.  Multiple test and certification labs shall exist for a specific technology. Certification and test specifications shall be open to those considering or seeking certification under reasonable terms.

| High | Several independent certification organizations are available to determine adherence to specification in implementation . Certification specifications are open. Test labs for technology characterization exist. |
| --- | --- |
| Medium | Few labs exist. The technology specification body might be self-certifying rather than use independent labs to do the certification. Certification specifications might not be open to those seeking certification. |
| Low | Unknown OR Certification bodies do not appear to exist or be indepedent in their assessment. |

## 1E) On the NIST Interoperability Roadmap

This lower level Systems Requirement is to compare communications technologies that have been announced as accpeted or considered on the NIST Interoperability Roadmap.  A significant panel of industry companies and experts have been leveraged to establish and govern acceptance into the list.   As the list was defined to catalyze Smart Grid technology uptake, its constituents are obviously relevant to Smart Grid use and should be considered with priority by Appliance Manufacturers.

| High | Technology has been approved by NIST/SGIP as interoperable for use in the Smart Grid. |
| --- | --- |
| Medium | Technology is being considered by NIST/SGIP for inclusion in the interoperable Smart Grid technology list. |
| Low | Unknown OR technology is not being actively considered by NIST/SGIP as an interoperable Smart Grid technology. |

## 1F) Supported and Vetted by SDO

This lower level Systems Requirement is to compare how communications technologies are  supported and vetted by an ANSI accredited standards definition organization (SDO).  Standards development organizations represent national interest in the long term interoperability of the underlying technology.  Use of an accredited SDO nearly establishes the required governance, openness, and community of relevance for a technology.

| High | An ANSI accredited standards development organization at the international or US national level has a standard that defines the technology or a supermajority of components thereof. |
| --- | --- |
| Medium | A standards development organization at the international or US national level has a draft standard that defines the technology or a supermajority of components therof. |
| Low | Uknown OR No standard of international or US national level exists for a supermajority of the technology. |

## 1G) Supported and Vetted by Commercial Alliance

This lower level Systems Requirement is to compare how communications technologies are supported and vetted by a commercial alliance whose purpose is to contribute, govern, advocate, and enforce adoption of features into a given technology specification in the commercial market.  Historically, SDOs do not market, certify, or directly develop technology, but govern and record the standard and its establishment in the most open and fair way.  Commercial alliances represent a healthy users group (typically vendor lead) for those seeking to add features and robustness to a technology, and are often used for certification oversight.

| High | A commercial specification development organization exists which is open to membership for development of the specification. |
| --- | --- |
| Medium | A commercial specification development organization may exist, but might not back the original specification or further revision of the specification. |
| Low | Unknown OR a commercial specification development organization does not exist |

# 1H) Capable of Delivering Smart Energy Services & Protocols

This lower level Systems Requirement is to compare  communications technologies on ability  to bear well known smart energy services and protocols.  While individual applications protocols have their own selection criteria, appliance manufacturers are seeking to select underlying PHY/MAC/NET technologies that are known or projected to be capable of bearing emerging smart energy application protocols without significant modification in the standards basis.  Key technological assumptions are generated by each protocol considered.  It is assumed that the best application protocols enable (interoperate independently of) the highest number of PHY/MAC technologies.

# 2) Accepted Technology in the Marketplace

This upper level Systems Requirement is to compare how communications technologies relevant to the markets in which they seek to participate.  The customer potentially has various smart-grid relevant (sub)networks pre-existing in the home.   Smart Grid is very likely to be leveraged by utilities or by appliances on behalf of consumers over advanced metering infrastructure, any home internet connection,  home electronics, demand response-purposed networks,  and other home automation networks.  If these networks already incorporate HAN technology to a significant degree, we consider the technology accepted in the marketplace.

## 2A) AMI / Electric Meter Vendor Use

This lower level Systems Requirement is to compare how communications shall be relevant to HAN technologies available today at AMI served gateways.

| High | Technology is offered by most AMI / electric meter vendors as an option. |
| --- | --- |
| Medium | Technology is offered by at least one AMI / electric meter vendor as an option. |
| Low | Unknown OR Technology is not offered by AMI / electric meter vendors as an option. |

## 2B) Home Internet Gateway Use

This lower level Systems Requirement is to compare how communications are relevant to HAN technologies available today at home internet gateways or other home routers suitable for connecting to such gateways.

| High | Technology is offered by most home Internet gateway vendors as an option. |
| --- | --- |
| Medium | Technology is offered by at least one home Internet gateway vendor as an option. |
| Low | Unknown OR Technology is not offered by home Internet gateway vendors as an option. |

## 2C) DR Gateway Use

This lower level Systems Requirement is to compare how communications are relevant to HAN technologies available today in special-purpose demand-response gateways.

| High | Technology is offered by most demand response gateway vendors as an option. |
| --- | --- |
| Medium | Technology is offered by at least one demand response gateway vendor as an option. |
| Low | Unknown OR Technology is not offered by demand response gateway vendors as an option. |

## 2D) Protocol Used by Consumer Electronics

This lower level Systems Requirement is to compare how communications are relevant to HAN technologies available today in consumer electronics.

| High | Technology is offered by many consumer electronics products. |
| --- | --- |
| Medium | Technology is offered by at least one consumer electronics vendor as an option. |
| Low | Unknown OR Technology is not typically offered by consumer electronics vendors. |

## 2E) Other HAN Installed Base Vendor Use

This lower level Systems Requirement is to compare how communications are relevant to other HAN technologies deployed today in the market such as home automation.

{  I think 2D is redundant of 2E and should be deleted  }

| High | Technology is used widely in the installed HAN communications technology base. |
| --- | --- |
| Medium | Technology is used seldomly in the installed HAN communications technology base. |
| Low | Unknown OR Technology is not typically used in the installed HAN communications technology base. |

# 3) Acceptable Cost Trade-offs

This upper level Systems Requirement is to compare how communications technologies minimize cost and maximize performance to the consumer.  There are important  cost trade-offs in the market available between station element cost, bandwidth, latency, and power consumption.  Each of these elements is in tension with the others in its limit.   Elements that overlap other areas were not necessarily included here (e.g. single station home coverage).

## 3A) Low Cost Per Physical Station

This lower level Systems Requirement is to compare how communications technologies have a low perceived cost to use in a product relative to the performance they offer an appliance.  Cost is generally most easily compared in terms of chipset cost, however it is wise to consider the cost due to the appliance communications technology wholistically. Further, though critical to individual manufacturers, exact pricing is irrelevant to differentiating technologies.  Communications technologies can be easily differentiated by their market perceived cost class.  For example, in todays market, low cost premises network communications chips are often considered to be < $5 US, medium < $10 US, and high anything beyond those numbers.  Such numbers vary widely historically and are to be held at arms length, though appliance customers will always prefer a lower price. There are ultra-low cost HAN targeted technologies available as well (e.g. falling below $1 US), but such technologies almost always lack interoperability, security, bandwidth, latency, and other key mechanisms for integrating appliances with smart grid.

| High | Cost of individual chipset is perceived as low to use in a product relative to the performance they offer an appliance. |
| --- | --- |
| Medium | Cost is perceived as medium to use in a product relative to the performance they offer an appliance. |
| Low | Unknown OR Cost is perceived as high to use in a product relative to the perfomance they offer an appliance. |

## 3B) Sufficient Data Bandwidth Available

This lower level Systems Requirement is to compare how communications shall have sufficient data bandwidth available to carry current appliance targeted energy application protocols.  Bandwidth is required to carry energy services and protocols to appliances. Energy services typically contain control and information signals centered around load control, pricing, consumption, and messaging.  Control bandwidth is estimated to become reasonably useful at 10 kb/s PHY rate.  Interactive bandwidth between a single pair of network stations is estimated to begin to be highly useful at bandwidths greater than 32 kb/s PHY rate. In situations where a network is expected to scale to distances beyond a single link, mesh architectures or other physically routed networks can require higher bandwidth available so that a station has enough bandwidth to act as a router without impacting scalability.  Room to grow beyond today’s applications into future application requirements for energy services is estimated to vary from 60 kb/s to 1 Mb/s PHY RATE.  It is hard to justify the potential future applications bandwidths without discussing  specific appliances and their interfaces.

| High | Technology’s bandwidth is more than sufficient to carry current appliance targeted energy application protocols and scales to multiple endpoints simultaneously. ~> 32 kb/s |
| --- | --- |
| Medium | Technology’s bandwidth is sufficent to carry current appliance targeted energy applications to all endpoints individually. ~>10 kb/s |
| Low | Unknown OR Technology’s bandwidth is below minimum expectation to carry appliance targeted energy applications. |

## 3C) Sufficient Data Latency Available

This lower level Systems Requirement is to compare how communications technologies shall have sufficient latency to deliver current appliance targeted energy appliactions protocols.	Link latencies higher than 100 ms do not support all current energy service applications. Link latencies between 100 ms and 20 ms are sufficient for appliance targeted energy applications.  Link latencies lower than 20 ms are less than the minimum required to support current and future appliance latency requirements.

| High | Technology’s latency is less than minimum required to support current appliance targeted energy applications. ~< 20 ms |
| --- | --- |
| Medium | Technology’s latency is sufficient to support current appliance targeted energy applications. < 100 ms |
| Low | Unknown OR Technology’s latency is higher than minimum required to support current appliance targeted energy applications. |

## 3D) Energy Proportional Power Consumption

This lower level Systems Requirement is to compare how communications technologies consume energy proportional to/from the amount of data sent and the range of transmission to a remote station.  Technologies should ideally have standard ways to enable energy proportional power consumption and to use them. It is expected that low idle power modes and scalable transmission power may be used to achieve this.  Reducing transmission power to the minimum required should reduce mutual interference at dense premises, helping with network scalability.  Appliances in EU and US have now categorized the limit and measure respectively the standby power of products.    This is the power measured when the product is idling.  This category compares average power over a period of time (24 hrs?) including hourly price updates.

| High | Technology idle power is relatively low, receiver power consumption is proportional to receive duration, and transmit power consumption is proportional both to transmit duration and minimum strength required to reach the remote station. |
| --- | --- |
| Medium | Technology idle power is moderate, receiver power may be proportional to receive duration, and transmit power consumption probably not proportional |
| Low | Unknown OR likely not proportional to media use |

4) Easy to Provision / Plug-and-Play Appliances

This upper level Systems Requirement is to compare communications technologies the  easy to provision. Appliances are currently expected to function once properly physically installed.   Appliance communications elements shall require a minimal number, with a preference for zero, of steps to get installed. Building the link from the appliance to its first destination shall be self explanatory to the customer.  Media extension and change options shall be available to the customer for remediation of building materials such as repeating and bridging.   Security is a must in modern networking technologies, security credential provisioning and secure joining shall bepossible, but require a minimum  number, with a preference for zero, of steps to get connected.

## 4A) Endpoints Easy to Join to a Network

## This lower level Systems Requirement is to compare communications technologies the ease with which the appliance customers can join the appliance to a premises network.   Specifically, underlying communications mechanism shall enable independent appliance and other network user interfaces to establish a connection with a minimum of interacitons.  Connecting the communications element of the appliance to a network shall require aminimum of effort on the part of the customer.

| High | Connection of an endpoint's communications element is trivial to accomplish by a customer to a premises network requiring only deictic gestures such as near-simultaneous button presses, physical token exchange, collocation, or any other near-automatic establishment of joining. |
| --- | --- |
| Medium | Connection of appliance to premises network is more involved; customer may have to manually indicate logical network membership by say entering unique identifiers at one or both endpoints via unspecified user interface. Minimal configuration to media may be required. |
| Low | Unknown OR Endpoints are challenging to get configured, may require customer to have a computer or purpose specific manipulator to establish correct operation. Configuration likely required beyond logical network group indication. |

## 4B) Secure Joining to Network Easy

This lower level Systems Requirement is to compare how communications technologies enable easy initial establishment of security for the connection from the appliance communications endpoint to the premises network.  Reducing configuration overhead due to security will broaden the market and improve chances customers can take advantage of security for the appliance to premises network.

| High | Security commissioning of the appliance communications does not necessarily require additional steps beyond initial join/connection or other logical grouping of the network. |
| --- | --- |
| Medium | A more involved security comissioning of appliance to premises network than the initial join, customer may have to enter or manipulate security tokens at one or both endpoints seperate from any grouping identifiers. |
| Low | Unknown OR Endpoints security is challenging to get configured, may require customer to have a computer or purpose specific manipulator to establish security configurations. |

## 4C) Integrated Support for PHY Extenders / Repeater Support

This lower level Systems Requirement is to compare how communications technologies inherent support for logical link (station-to-station) extension via repeating or other PHY-extending means.  While it is envisioned that a single node in the home network would have sufficient mechanisms to maintain its provisioned bandwidth and latency at ranges covering a whole home, this is not always possible.  Appliances may be large and unable to be relocated while potential customers may already have a well-established internet connection.  Construction materials, multipath, interference, and other attenuation of the signal on the media may require the customer to give up or seek alternative comms provisioning means.  The market needs a reliable well-understood way to deal with assistance in connecting the appliance to the first element of the premises network under these conditions.

| High | The technology has architectural support for PHY extension / repeaters. |
| --- | --- |
| Medium | The technology may have after-market support for PHY extension / repeating. |
| Low | Unknown OR The technology does not have PHY extension / repeating. |

## 4D) Integrated Support for Change of PHY / Bridging Support

This lower level Systems Requirement is to compare how  communications technologies inherent support for logical link (station-to-station) and network extension via bridging or other PHY-changing means.  While it is envisioned that a single node in the home network would have sufficient mechanisms to maintain its provisioned bandwidth and latency at ranges covering a whole home, this is not always possible.  Further, the premises network is sometimes a different physical media than the appliances provisioned media.  Appliances may be large and unable to be relocated while potential customers may already have a well-established internet connection.  Construction materials, multipath, interference, and other attenuation of the signal on the media may require the customer to give up or seek alternative comms provisioning means.  The market needs a reliable well-understood way to deal with cases where wired, wireless, or special purpose wired appliance communications technologies need to be changed to avoid a limiting feature of the premises or integrate with a specific media type of prexisting premises network.

| High | The technology has architectural support for change of PHY / bridging. |
| --- | --- |
| Medium | The technology may have after-market support for change of PHY / bridging. |
| Low | Unknown OR The technology does not have change of PHY / bridging. |

## 4E) Supports Provisioning User Feedback

This lower level Systems Requirement is to compare how communications technologies provide or enable provisioning and connection status feedback.  The technology shall be able provide notification of network joined, join success/failure, failure reasons and conditions, signal strength, and other key potential joining UI information requirements.  Ideally, any established network side of the join shall be able to detect the presence of an appliance seeking to join and be capable of enabling similar feedback.   The key to enabling a good customer experience is enabling the first experience with the communications link to be as simple and user friendly as possible.  Having ability to access provisioning user feedback at the communications layer enables innovation of easy to use joining interfaces and procedures.

| High | Technology can enable provision of diagnostic feedback to the user under most failure or success situations for most commissioning / registration / enrollment situations. |
| --- | --- |
| Medium | Technology can provide user with status feedback only under some success or failure scenarios for commissioning / registration / enrollment situations. |
| Low | Unknown OR Technology does not intentionally have mechanisms to enable provisioning user feedback. |

# 5) Minimizes Enabling Infrastructure

This upper level Systems Requirement is to compare how communications technologies minimize the requirement for additional enabling infrastructure.  Appliance communications shall use a minimum of power.  Appliance communications elements shall require a minimum of additional devices to provide maximum coverage area of the home.  Appliance communications elements shall require a minimum of additional cabling the beyond basic appliance install to establish a communications link. Minimizing enabling infrastructure helps the consumer to plug and play.  Seeking to minimize additional resources will enable the consumer and consumer markets to adopt, support, leverage, or offer the appliance communications technology expediently.

## 5A) Enables Low Power Consumption

This lower level Systems Requirement compares how communications technologies meet or exceed market and regulatory requirements for appliance power consumption. Regulatory compliance or enablement thereof is a limiting issue in a technology’s adoption.  Reducing power consumption due to communications media additionally enhances the appliance’s ability to add value to the customer and its use in any premises energy applications.  It is expected that every technology consumes power in a slightly different way, but that at the end of the day power consumption accountable to energy application communications must be sufficiently low to enable the appliance.  If sufficiently low power consumption can not be achieved it is also more likely that additional external devices will be required to establish communications, reducing value to the customer.

| High | Average expected power consumption due to communications technology is less than .3 W at full transmit duty cycle assuming no additional PA/LNA or other AFE loss. |
| --- | --- |
| Medium | Average expected power consumption is less than 1 Watts at full transmit duty cycle assuming no additional PA/LNA or other AFE loss. |
| Low | Unknown OR Average expected power consumption is greater than 1 Watts assuming no additional PA/LNA or other AFE loss. |

## 5B) High Single Station Coverage

This lower level Systems Requirement is to compare how commmunications technologies capablities of covering a large area of a typical premises from a single network station.  No one solution is capable of addressing all types of install situations possible at a premises.  Solutions are preferred that require fewer installed elements in the limiting case.  This preference for coverage is at odds with interference potential.  Other criteria such as coexistence mechanism and transmit power control represent the mitigations required to scale within and between homes separately.

| High | A single station using the technology is likely to cover large portion of a typical premises. |
| --- | --- |
| Medium | A single station using the technology is not likely to cover a large portion of a typical premises and repeaters or bridges likely to be required. |
| Low | Unknown OR Technology is likely to be limited to only a few rooms of premises for coverage. |

## 5C) Requires Minimal Additional Cabling

This lower level Systems Requirement is to compare how communications technology require no additional cabling beyond normal install for the appliance. Requiring additional cables will necessarily reduce the number of customers who are able and willing to leverage the communications module in the product for energy applications.

| High | Technology requires no additional cabling or connection beyond normal install for appliance to function as a network station. |
| --- | --- |
| Medium | Technology leverages existing premises wiring but likely requires additional cables, phase couplers, bypass couplers, or blocking filters to get fully connected to the premises for a significant set of premises. OR Technology is wireless and requires additional external antennae / wires to get connected to the premises network for a significant set of premises. |
| Low | Unknown OR Technology is like special purpose wired technologies such as USB, Ethernet, serial, etc. that would not typically have a point of interconnect available at appliance point of install for most premises. |

## 5D) Topology Minimizes Additional Stations

This lower level Systems Requirement is to compare how communications technology topologies can enable avoidance of additional stations to enable normal install for the appliance. Requiring additional stations will necessarily reduce the number of customers who are able and willing to leverage the communications module in the product for energy applications.   Further, additional stations contribute to the interference condition and potentially reduce available bandwidth.

| High | Technology requires no additional stations beyond those at the appliance and energy service protocol service point. Technology typically also supports repeating and routing if not meshing. |
| --- | --- |
| Medium | Technology may require additional stations but gains significant coverage at their installation. |
| Low | Unknown OR Technology is like special purpose wired technologies such as USB, Ethernet, serial, etc. that gain no broad coverage advantage from addition of stations. |

## .

# 6) Technology Supply Chain Exists

This upper level Systems Requirement is to compare how communications technologies have an established supply chain.  The appliance communications technology market should be able to supply everything an appliance manufacturer needs to get started with and integrate the technology into their products and scale to production.

## 6A) Integration Talent Exists

This lower level Systems Requirement is to compare how communications technologies  have a well-established integration talent base. Most brand new technologies or technologies under development represent specialist skill sets. Ideally the design and integration talent base is well established.

| High | It is easy to find development support and development talent for the technology. |
| --- | --- |
| Medium | The technology appears to have a sole source for development talent. |
| Low | Unknown OR Technology heavily proprietary or technology under development, integration talent is defacto. |

## 6B) Integration Tools Exist

This lower level Systems Requirement is to compare how communications technologies have a well-established integration tools base available to the market. Most brand new technologies or technologies under development have limited tool sets, though some older technologies can too. Ideally the design and integration tools base is well established.

| High | It is easy to find development tools for the technology, tools are likely available from multiple vendors. |
| --- | --- |
| Medium | There is a sole source for development tools. |
| Low | Unknown OR Technology is heavily proprietary or technology under development, tools likely don’t exist. |

## 6C) Part Availability

This lower level Systems Requirement is to compare how communications technologies have parts and software readily available to the market for development and integration of appliance targeted energy service communications.

### 6Ca) Multiple Silicon Implementers Identified

This lower level Systems Requirement is to compare how communications technologies have multiple silicon vendors in the market to promote a higher reliability of the supply chain.

| High | The technology has multiple silicon vendors and the vendors are unambiguously diverse. |
| --- | --- |
| Medium | The technology has more than one silicon vendor, multiple vendor relationships may be ambiguous. |
| Low | Unknown OR The technology has only a sole source for silicon. |

### 6Cb) Silicon Availability

This lower level Systems Requirement is to compare how communications technologies  have silicon available to be purchased now.   Emerging appliance communication technologies should have silicon available between 6 mo and 1 year.

| High | The technology has silicon available now. |
| --- | --- |
| Medium | The technology will have silicon available within 6 mo. |
| Low | Unknown OR The technology will have silicon available within a year or greater. |

### 6Cc) Software Availability

This lower level Systems Requirement is to compare how communications technologies have software available (if required) to support platform integration. Emerging apliance communications technologies should have software available between 6 mo and 1 year.

| High | software or firmware is available for the silicon now |
| --- | --- |
| Medium | The technology will have software or firmware available within 6 mo. |
| Low | Unknown OR The technology will have software or firmware available within a year or greater. |

# 7) Future Proof

This upper level Systems Requirement is to compare how communications technologies have mechanisms to extend longevity of relevance of the communications technology in the market.  Appliances may be purchased individually. Customers should be able to buy new appliances thoughout their existing appliance’s lifecycle without requiring a redployment of communications technologies already deployed and configured at the premises.

## 7A) Backward Compatibility

This lower level Systems Requirement is to compare how communications technologies have mechanisms and/or established precedent to support prior revisions of the technology.  As families of technologies emerge, long term interoperability of early revision technology is highly value to the consumers and manufacturers seeking to stay current with emerging product requirements and capabilities.

| High | Technology is fully backward compatible with prior revisions. |
| --- | --- |
| Medium | Technology is partially bacward compatible with prior revisions or only some prior versions. |
| Low | Unknown OR Technology is not backwards compatible or no prior revisions exist such that backwards compatibility is not established. |

## 7B) Firmware Upgrade Exists

This lower level Systems Requirement is to compare how communications technologies have a specified firmware upgrade mechanism to support field upgrades of the communications element.  Communications firmware revisions might be required to extend the longevity or purpose of the platform.

| High | The technology’s specification has a firmware upgrade mechanism available. |
| --- | --- |
| Medium | The technology does not specify a firmware upgrade mechanism, but it is likely that a proprietary firmware upgrade mechanism exists for a specific vendor’s platoform. |
| Low | Unknown OR The technology does not specify a firmware upgrade mechanism and it is not likely the vendors offer one. |

# 8) Handles Interference and Noise

This upper level Systems Requirement is to compare how communications techonolgies have appropriate means for handling interference and noise.   The technology should be designed to scale to dense environments.  The technology should have a means of sharing its spectrum with other technologies.  The technology should have mechanisms to reject narrowband and transient noise sources. Finally, the technology should have transmit power control implemented to minimize its own radiated interference.

## 8A) Scales to Multiple Dwelling Unit Environments

This lower level Systems Requirement is to compare how communications technologies have the means to scale to dense residential environments such as multi-dwelling units (MDU).  A key issue in MDU scaling is the ability to get a sufficient number of network stations to operate properly well within transmission range of each other.  Since the customer is expected to install and operate any of these technologies, they will not be able to deal with interference sources from other units of an MDU.  The technology would ideally be sufficiently scalable and handle scaling automatically.

| High | The technology has a sufficient scalability strategy to support ad-hoc multiple dwelling unit installations. |
| --- | --- |
| Medium | The technolagy has sufficient scalability to support some multiple dwelling units, but does not likely scale to larger units. |
| Low | Unknown OR T echnology does not have good scaling properties to multiple dwelling units. |

## 8B) Coexistence Mechanism Used

This lower level Systems Requirement is to compare how communications technologies have an automatic means for coexistence with other users of the spectrum of deployment.  Coexistence enables the customer to buy and install products of their choice to provision other networks in their home without significant consequence due to the type of technology selected on appliance communications networks.  Coexistence mechanisms that are standardized are preferred.  Mandatory use of coexistence standards by a technology is preferred.

| High | Technology specifies a mandatory coexistence mechanism. The mechanism is standardized. |
| --- | --- |
| Medium | Technology specifies a coexistence mechanism. The mechanism may not be mandatory, the coexistence mechanism may only work within one alliances technology offering. The coexistence mechanism may only address certain additional technologies outside its own alliance. The mechanism may not be mandatory. |
| Low | Unknown OR No coexistence mechanism is used by the technology. Coexistence is assumed merely by avoidance. |

## 8C) Supports Transmit Power Control

This lower level Systems Requirement is to compare how communications technologies have a means for transmit power control. This enables appliances to limit the interference they create to the minimum required to communicate. If power control level is not controllable, the technology should at least be able to turn off its transmitter between transmissions.

| High | The technology has a means to scale its TX power emitted. |
| --- | --- |
| Medium | The technology has an ability to turn off its TX when not in use. |
| Low | Unknown OR The technology does not have transmit power control. |

## 8D) Supports Spectrum Spreading

This lower level Systems Requirement is to compare how communications technologies have a means for rejecting narrowband jammers . Jammers may be present in the target install environment. Customers may not be able to see or understand the interference issues present.  The technology should have a spectrum spreading means to facilitate bypass of these jammers.

| High | The technology has a means by which to reject narrowband jammers. |
| --- | --- |
| Medium | The technology may not be able to ignore narrowband jammers, but may be able to avoid or adapt them eventually. |
| Low | Unknown OR The technology does not have a means by which to deal with narrowband and transient jammers |

# 9) Easy System Operation

This upper level Systems Requirement is to compare how communications technologies enable easy system operation by building stable and reliable links and enabling appropriate user interfaces to be built around the technology.

## 9A) Enables Stable and Reliable Links

This lower level Systems Requirement is to compare how Appliances shall have mechanisms to enable construction of stable and reliable links. Technologies should have good link budgets, good receiver error margins, spectrum spreading, and scalable modulation means to offer stable links.  Technologies that can offer a stable link will be far more satisfying to the customer.

| High | Technology has good receiver error margins, modulation options, spread spectrum, and wide link budgets. |
| --- | --- |
| Medium | Technology may have some of the following: good receiver error margins, modulation options, spread spectrum, and wide link budgets. |
| Low | Unknown OR Technology has narrow link budgets, no ability to reject narrowband and transient jammers, no spread spectrum. |

## 9B) Provides Link State Feedback

This lower level Systems Requirement is to compare how communications technologies enable feedback on link state to the customer.  Consumers will not necessarily understand if or how to repair the underlying communications technology under conditions where a link fails.  Its up to the manufacturer to build good user interfaces, but the underlying technology needs to be able to communicate link state to the user interface.

| High | The technology can enable UI construction to indicate what is wrong / right with a link, how much, and what can be done to correct, and the current link up/down status. |
| --- | --- |
| Medium | The technology can enable UI construction to indicate the current link up/down status. |
| Low | Unknown OR The technology is poor at enabling indictation of link up/down status or other link state feedback. |

# 10) Easy to Maintain

This upper level Systems Requirement is to compare how  communications technologies are easy to maintain.  It is key that day-to-day / month-to-month use of the appliance communications technology by the consumer shall not require attention of the user towards the communications technology itself.

| High | The technology requires minimal user interaction to maintain. |
| --- | --- |
| Medium | Technology may require user service to maintain. |
| Low | Unknown OR The technology requires service call or service installer to maintain. |

# 11) Easy to Replace

This upper level Systems Requirement is to compare how communications technologies are easy to replace.   Should elements of the communications capability obviously break, the technology should be designed so consumer should be able to replace them easily, service the device,  or find an installer or repair service to service the device.

| High | The technology is designed for consumer replacement. |
| --- | --- |
| Medium | The technology may require user service to replace. |
| Low | Unknown OR The technology requires service call or service installer to replace. |

# 12) Robust Messaging Capability

This upper level Systems Requirement is to compare how communicaitons technologies be capable of robust messaging.  Energy services potentially provide control stimuli to both the appliance and the consumer.  Messages must be delivered correctly and in a timely fashion.

## 12A) Supports and Enforces Quality of Service (QoS) Delivery

This lower level Systems Requirement is to compare how communications technologies support and be capable of enforcing quality of service (QoS) across communications links.   Quality of service enables prioritization of transported messages. Grid-relevant events shall generally be infrequent, but may be desired to be transported with priority.  This technology becomes especially important should the consumer attach appliances to existing premises networks.

| High | T echnology has inherent, standards based, support for quality of service delivery of information. |
| --- | --- |
| Medium | T echnology has ad-hoc support for quality of service delivery of information, or limited enforcement. |
| Low | Unknown OR T echnology does not offer quality of service information delivery. |

## 12B) Offers Error Correcting Codes

This lower level Systems Requirement is to compare how communications technologies shall support error detection and error correction coding.  The technology shall make use of any codes to increase the reliability of appliance targeted energy services. Typical codes or pre-coding mechanisms at least detect burst and random error types resulting in retransmission attempts.  Good coding systems will use some additional portion of the bandwidth to transmit codes capable of correcting the errors.

| High | The technology has useful error correcting codes, handling burst and random errors. |
| --- | --- |
| Medium | The technology has error detection abilities, may be able to correct limited errors. |
| Low | Unknown OR The technology does not offer sufficient error detection and correction codes. |

# This upper level Systems Requirement is to compare how communications technologies are easy to replace.   Should elements of the communications capability obviously break, the technology should be designed so consumer should be able to replace them easily, service the device,  or find an installer or repair service to service the device.

# 13) Affordable to Install

This upper level Systems Requirement is to compare how communications technologies affordablity to install.  Ideally, installing the appliance in any traditional sense is equivalent to installing the communications.  Installation steps should not allocate additional cost to the consumer.

| High | Communications technology likely requires no additional monetary cost for customer to install. |
| --- | --- |
| Medium | Communications technology likely requires limited additional monetary cost for customer to install. |
| Low | Unknown OR Technology can require significant investment by the customer under certain conditions. |

# 14) Sufficient Enabling Security Mechanisms

## 14A) Link Encrypted

This lower level Systems Requirement is to compare how communications technology have  sufficient link encryption mechanism.  It should be possible for consumers to use link encryption on appliance network links to protect link confidentiality and integrity.  AES-128-CCM or better encryption modes are recommended mechanisms available in the market.  Some technologies offer link encryption, but may have substandard or obsolete methods.  Some technologies won’t offer encryption mechanisms.

| High | Technology has the ability to encrypt all point-to-point links indepedentely using AES-128-CCM or better. |
| --- | --- |
| Medium | Technology may have the ability to encrypt all point-to-point links indepedentely but may have insufficient cryptography or cryptography mode strength. |
| Low | Unknown OR Technology does not encrypt links independently. |

## 14B) Network Encrypted

This lower level Systems Requirement is to compare how communications technology  have  sufficient network encryption mechanism.  Network encryption differs from link encryption in that nodes on a single logical network are logically seperated by encryption from nodes on any other network, independent of any individual link encryption or link technology.  It should be possible for consumers to use network encryption on appliance networks to protect network level confidentiality and integrity.  AES-128-CCM or better encryption modes are recommended mechanisms available in the market.  Some technologies offer network encryption, but may have substandard or obsolete methods.  Some technologies won’t offer encryption mechanisms.

| High | Technology has the ability to encrypt all networks independently using AES-128-CCM or better. |
| --- | --- |
| Medium | Technology may not have the ability to encrypt networks independently, may not have sufficient strength cryptography. |
| Low | Unknown OR Technology does not encrypt networks independently. |

## 14C) Endpoints Mutually Authenticatable

This lower level Systems Requirement is to compare how communications technologies have a mechanism to enable mutual authentication between any two network stations.  To help protect consumers from rogue or outside control, any station on or joining a logical network should be able to mutually authenticate any other node with which it wishes to communicate.  In some cases, technologies will only offer one-way or group authentication only.  Technologies that don’t support authentication potentially place the customer and their appliances at risk.

| High | Technology has ability to mutually authenticate between network stations. |
| --- | --- |
| Medium | Technology has at least one-way authentication. |
| Low | Unknown OR Technology does not have the ability to authenticate stations. |

## 14D) Supports PKI

This lower level Systems Requirement is to compare how communications technologies have a mechanism to support Public Key Infrastructure (PKI) style station identity certification for protection of station and network  integrity.  Ideally every station could have a unique certificate that is linked to and supports deployed network security policies.

| High | Technology has ability to enable or use public key infrastructure. |
| --- | --- |
| Medium | technology has ability to enable or use public key infrastructure, use may be weak, limited, or improper. |
| Low | Unknown OR Technology does not have ability to enable or use public key infrastructure. |

## 14E) Supports PSK

This lower level Systems Requirement is to compare how communications technologies have a mechanism to support pre-shared key (PSK) style station identity certification.  Ideally a customer or other entity would be able to deploy a pre-shared key that is used to logically group network elements.

| High | Technology has ability to enable or use pre-shared keys. |
| --- | --- |
| Medium | Technology has ability to enable or use pre-shared keys, use may be weak, limited, or improper. |
| Low | Unknown OR Technology does not have ability to enable or use pre-shared keys. |

## 14F) Supports Logical Separation

This lower level Systems Requirement is to compare how communications technologies have a mechanism to logically separate one customer network from another.  At a minimum network traffic should be separatable by name or address. Ideally, cryptographic enforcement of logical separation would be possible.

| High | Technology has ability to logically separate communications, networks, links and enforce this via cryptography. |
| --- | --- |
| Medium | Technology has some ability to logically separate communications, networks, links but may not enforce this via cryptography. |
| Low | Unknown OR Technology has no ability to logically separate communications, networks, link channels. Definitely doesn't use cryptographic enforcement. |