Appliance Communications Technology Evaluation Outcome

This document describes the top three Smart Grid communications technologies evaluated and under consideration by appliance manufacturers for use in Smart Grid integration.  Integration architectures and evaluation criteria for selection of these technologies have been defined separately.  Covered within are brief technical descriptions of ZigBee, Wi-Fi, and HomePlug Green PHY followed by side-by-side description of their performance in the specific study evaluation criteria.

Contents

Individual Technology Descriptions	3

ZigBee	3

Physical Layer	3

Reliability	3

Networking	4

Security	4

Power Consumption	4

Low Power Embedded Wi-Fi	5

Physical Layer	5

Reliability	5

Networking	6

Security	6

Power Consumption	6

HomePlug Green PHY	7

Physical Layer	7

Reliability	7

Networking	7

Security	8

Power Consumption	8

Overall Technology Evaluation Outcome	9

Proven Interoperable PHY/LINK/NET	9

Accepted Technology in the Marketplace	9

Acceptable Cost Trade-offs	9

Easy to Provision / Plug-and-Play Appliances	10

Minimizes Enabling Infrastructure	10

Technology Supply Chain Exists	11

Future Proof	11

Handles Interference and Noise	12

Easy System Operation	12

Easy to Maintain	12

Easy to Replace	12

Robust Messaging Capability	13

Affordable to Install	13

Sufficient Enabling Security Mechanisms	13

Conclusions	14

# Individual Technology Descriptions

## ZigBee

ZigBee is a wireless mesh networking technology and application profile set that leverages, enhances, and extends the standards base. It offers IEEE 802.15.4 radio links, 6LoWPAN IPv6 network delivery, standards based mesh routing and best of breed standards based security for very low power and chip cost hovering around $5 in volume.  ZigBee has been rearchitected over the past 3 years onto an open standards basis specifically to support Smart Grid application requirements based on utility and vendor guidance.  As a result, IEEE 802.15.4 radios have been incorporated into many electricity meter Advanced Metering Infrastructure (AMI) deployments to help utilities offer Smart Grid services into the customer home.  Some AMI meters use ZigBee as their primary field area network system due to mesh robustness in dense urban areas.  Recent trends include rapid adoption of ZigBee in the Home Area Networking (HAN) market for home automation and controls due to simplicity and low cost.

### Physical Layer

ZigBee’s wireless transmit and receive capability are IEEE 802.15.4-2006 implementations, including subsets of the PHY and MAC specifications. Historically, ZigBee devices may be certified to operate across any of 16 adjacent non-overlapping 5 MHz channels overlaid on the 2.4 GHz ISM band starting at f_c=2.405 GHz and preceding to 2.480 GHz. Recently, certification options have been extended to other profiles of 802.15.4-2006 including 900 MHz.  ZigBee signals transmit and receive using a 2MHz signaling bandwidth centered in one of the 5 MHz channels.  ZigBee avoids potential narrowband interference by using Direct Sequence Spread Spectrum (DSSS) coding to spread energy around potential interference sources within band.  ZigBee operating in 2.4 GHz uses a modulation technique called Offset Quadrature Phase Shift Keying (OQPSK) to modulate chips or sets of bits represented as complex symbols onto the carrier frequency.    PHY rates for IEEE 802.15.4 following coding steps is approximately 250 kb/s , but higher layer protocols including those at the MAC layer and those added by ZigBee are reduce this bandwidth further.   Link latencies are typically 21 ms – 32 ms per hop plus any node delay in low interference conditions.

Minimum receiver sensitivity is established by IEEE 802.15.4 as -85 dBm, but in practice chips are available that approach -100 dBm receiver sensitivities without addition of low noise amplifiers (LNA).  Transmit power is implementation and design specific, but must conform to at least FCC Part 15 guidance in the US.  ZigBee was originally intended as a 0 dBm transmit power technology that would rely more heavily on its higher layer meshing capability, but its fields of application have caused the chips built to vary around that number.  Transmit power available at the chip typically range between -17 dBm and +5 dBm.  ZigBee modules are available on the market with integrated power amplifiers (PA) that can boost the signal to +20 dBm.  The IEEE 802.15.4 PHY format and some analog front ends and chipset can support antenna diversity sufficiently to see benefit, offering some enhancements for addressing multipath environments and line of sight occlusion cases.

### Reliability

ZigBee’s link layer manages reliable transmission between two adjacent stations.   Techniques include Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA) to attempt to avoid intentional interference, frame check sequences (FCS) to catch modulation errors, and acknowledged frames with retransmission of unacknowledged frames to help guarantee delivery. Application layer network channel reallocation schemes allow the network coordinator to move the network should interference continue to be a challenge.  The ZigBee protocols additionally have Link Quality Indication (LQI) and Receive Signal Strength Indication (RSSI) mechanisms to enable stations to understand their performance relative to environmental conditions.

### Networking

ZigBee has recently adopted IETF 6LoWPAN, an IPv6 over IEEE 802.15.4 technology and family of RFCs.  Adaptation to/from 6LoWPAN is performed at ZigBee gateways to allow any IPv6 enabled consumer product to interact with an adjacent ZigBee network.  At least one gateway class device is required at the edge of a ZigBee node to control and secure the network.  ZigBee nodes will have mechanisms to serve TCP and/or UDP connections, at a minimum, to further this goal.  ZigBee handles the details of mesh routing both to wireless devices within the connected network and across any other connected networks should they exist.   ZigBee’s meshing algorithm is the key to its price/performance argument.  When only two nodes are present, ZigBee performs similar to most other point-to-point and star topologies that would use DSSS and phase shift keying.  As more than two stations capable of routing are added within proximity of each other, a ZigBee network becomes far more robust than a non-mesh network. So-called path diversity enables communications to route around obstacles and multi-path interference sources, effectively reaching beyond the link budget to the destination.

### Security

ZigBee uses a centralized root of trust model to identify and mutually authenticate nodes joining the network.  ZigBee uses Elliptic Curve Cryptography (ECC) certificates to represent identity at nodes in the network; a NIST approved and standardized mechanism for embedded device asymmetric cryptography.  The IETF standard Extended Authentication Protocol (EAP) will be used to authenticate communications nodes joining using their certificates.  IETF standard Transport Layer Security (TLS) is used to manage secure sessions and the Advanced Encryption Standard (AES) 128-CCM mode encryption will be used to encrypt communications both across station to station links and for applications across ZigBee networks.  ZigBee security has recently been harmonized with these very typical standards to enable security interoperability with any other adjacent networks.

### Power Consumption

ZigBee is inherently low power and supports various architectures including battery powered end node devices.  Power consumed due to a ZigBee station can easily fall below the half watt barrier in home area networking, with clever implementations falling low enough to be powered for 6 months or more with enough responsiveness to still be relevant to smart grid implementations.   ZigBee stations typically consume power proportional to their time awake as implemented by a given vendor, with typical power consumption being spent more on receive than transmit for applications not using a PA and transmitting with low duty cycle.

## Low Power Embedded Wi-Fi

Wi-Fi is a market dominant Home Area Networking (HAN) standard with great relevance to the consumer and any HAN they may already have a variant of installed in their premises.  Wi-Fi is standardized under the IEEE 802.11 group with many variants in existence.  The work in Wi-Fi most relevant to Smart Grid falls in the camp of 802.11n.  Specifically Low Power Embedded Wi-Fi (LPEW) implementations of the 802.11n standard are relevant to appliances addressing the cost and power consumption concerns of appliances.  Recent variants of LPEW 802.11n are being adapted to support Smart Grid application delivery.  As a result, IEEE 802.11 radios are being incorporated into electricity meter Advanced Metering Infrastructure (AMI) deployments in increasing number to help utilities offer Smart Grid services into the customer home.  Some AMI meters also use Wi-Fi as their primary field area network system due to availability of metropolitan Wi-Fi.

### Physical Layer

LPE Wi-Fi designs use IEEE 802.11n for their wireless transmit and receive capability, offering a subset of the optional bands and modulators found in a typical consumer Wi-Fi product.  LPE Wi-Fi is currently typically being deployed in the 2.4 GHz ISM band.   5.8 GHz variants would be less desirable for Smart Grid applications due to increased attenuation in that band for home environments.    LPE Wi-Fi typically avoids potential narrowband interference by using Direct Sequence Spread Spectrum (DSSS) coding to spread energy around potential interference sources within band.  Some variants currently offer advanced Orthogonal Frequency Division Multiplexing (OFDM) spreading modes as well. LPE Wi-Fi stations are configurable to operate in one of 11 overlapping 22 MHz channels with 5 MHz center separation.   The modulation means supported vary too widely for useful report herein.  As the goal is to save power and reduce host overhead, most solutions offer one or two of lowest data rate modulators for DSSS or OFDM modulation only.  Data rates for LPE Wi-Fi using DSSS are 1-2 Mb/s and 6 Mb/s for OFDM.  Such data rates are nearly excessive for appliance Smart Grid needs, but are the minimum requirement to be directly interoperable with any existing consumer Wi-Fi infrastructure. In practice 802.11n can offer up to 4ms or slower latency in low interference short range applications.

In practice, receiver sensitivity for Wi-Fi is approximately -92 dBm to -96 dBm without use of a Low Noise Amplifier (LNA) at the lower data rates common in LPE Wi-Fi. Transmit power is implementation and design specific, but must conform to at least FCC Part 15 guidance in the US.  LPE Wi-Fi chips are known to exist that can offer between zero dBm and +12 dBm transmit power without use of an external Power Amplifier (PA). LPE Wi-Fi supports 1X1 and antenna diversity designs today and 3x1 MIMO designs are very near to market.

### Reliability

Wi-Fi’s link layer manages reliable transmission between two adjacent stations. Techniques include Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA) to attempt to avoid intentional interference, frame CRCs to catch modulation errors, and acknowledged frames with retransmissions of unacknowledged frames to help guarantee delivery.  Further, Wi-Fi uses Forward Error Correction (FEC) a technique which proactively attempts to avoid retransmissions.   A Wi-Fi implementation can have Receive Signal Strength Indication (RSSI) and packet error / loss accounting mechanisms to enable stations to understand their performance relative to environmental conditions.

### Networking

Wi-Fi excels in that it offers an 802.2 compliant transport, which easily integrates with existing customer Ethernet and Wi-Fi infrastructure.   IPv6 and standard routing mechanisms are used directly and without abbreviation or adaptation.  Wi-Fi is a star topology network requiring that one Wi-Fi station acts as the access point for all others on the wireless segment.  Wi-Fi also supports wireless bridging (WDS) such that additional APs may be deployed to extend the coverage area.  However, bridging support comes at the expense of additional interference and additional AP class devices.

### Security

Wi-Fi uses a centralized root of trust model to identify and mutually authenticate nodes joining the network.  Wi-Fi can use asymmetric cryptography including Elliptic Curve Cryptographic (ECC) and RSA cryptographic certificates to represent identity at nodes in the network. ECC and RSA are NIST approved means. Wi-Fi supports use of 802.11i standard mechanisms for authentication directly. Via 802.11i, it can use the IETF standard Extended Authentication Protocol (EAP) to authenticate communications nodes joining using certificates. Wi-Fi offers WPA2 as its means for securing links. IETF standard Transport Layer Security (TLS) may easily be used over Wi-Fi to manage secure sessions. The Advanced Encryption Standard (AES) 128-CCM mode encryption is used to encrypt communications between stations and for Smart Grid applications.  Wi-Fi additionally has customer centered secure device join mechanisms offered under the Wi-Fi Protected Setup (WPS) portion of the standard.

### Power Consumption

Wi-Fi is historically not a low power technology.  However, recent innovations in targeting silicon for low power embedded systems have allowed development of low power endpoint nodes.  For some offerings, power consumed at a LPE Wi-Fi device can be achieved to fall just at the half watt barrier in home area networking.   With clever developer implementation, potentially trades in link budget, data rate, and robustness the power consumption appears to be capable of being driven far below half a watt for Smart Grid use case profiles. Wi-Fi stations typically consume power proportional to their time awake as implemented by a given vendor, with typical power consumption being spent more on receive than on transmit for applications not using a PA and transmitting with low duty cycle.  Wi-Fi does have extensions that require the AP to manage power consumption that theoretically enhance site-wide power performance without additional requirement for clever implementation.

## HomePlug Green PHY

HomePlug Green PHY is a high frequency broadband over power line communications (PLC) technology adapted specifically to meet US premises smart grid device requirements.  The Green PHY has been adapted from the market-proven HomePlug AV offering and retains PHYs, MACs, and network layer interoperability with HomePlug AV and P1901-fft with several enhancements.  Though not yet deployed, Green PHY is expected to be relevant to home energy markets due to its designed scalability, low projected cost, low power, high in-premises coverage and access, and other factors detailed below.

### Physical Layer

Green PHY operates across the 2-30 MHz band of the spectrum available on a powerline.  Green PHY consists of an OFDM based signaling layer with robust modulation.  Green PHY uses the wiring on the premises side of the transformer as a shared media bus.  On top of its PLC layer PHY/MAC technology sits an 802.2 compliant 802.3 Ethernet adaptation layer.  This layer is commonly used in existing products to deploy Ethernet and Wi-Fi over PLC.

Historically, HomePlug AV employs three robust modes of communication, called ROBO Modes, for several purposes, including beaconing, data broadcast/multicast communication, session setup, and exchange of Management Messages.  All ROBO Modes use QPSK modulation, along with a ½ rate Turbo Convolutional Code. The Green PHY uses only these modes and retains full interoperability with HomePlug AV.  Besides the robust nature of these modes, the further advantages of using only the ROBO modes for the Green PHY are as follows:  minimum impact on HPAV throughput,  interoperable with current HPAV and future IEEE P1901 solutions baselined on HPAV (the Green PHY will be certified by HomePlug as a profile of P1901), able to support multiple PHY Rates (3.8 Mbps, 4.9 Mbps, 9.8 Mbps), reuses the entire 2 – 30 MHz band, significantly reduces complexity and therefore cost of the AV PHY (e.g., FEC, AFE, DFE), and enables low power modes via reduced duty cycle.

### Reliability

Green PHY’s PHY layer is extremely robust, effectively extending access and coverage well across 24 VAC home HVAC transformers, 240 VAC main phase, and both 120 VAC phases without additional hardware or issue.   Green PHY is coexistent with other competing high frequency broadband PLC technologies through its incorporation of IEEE P1901 inter-system protocol (ISP). Green PHY’s MAC uses Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA) to attempt to avoid interference with other stations, frame CRCs to catch modulation errors, and acknowledged frames with retransmissions of unacknowledged frames to help guarantee delivery.  Further, Green PHY has very strong Forward Error Correction (FEC) at the PLC MAC layer.

### Networking

At the shared powerline media, Green PHY allows for the case of shared media for up to 128 premises per residential transformer with 32 devices per home transmitting.  In practice, it would be rare that 128 homes sharing a transformer would have full visibility of each other, and the practical limits for scalability are likely higher.  Additionally at the PLC PHY/MAC layer Green PHY adds support for repeating and routing as developed in IEEE P1901 to hide the details of automatic network construction for situations like extended range multi-dwelling unit scalability.   At the Ethernet layer, Green PHY behaves like Green PHY has inherent support for VLANs, TCP, IPv4, IPv6, and delivery of other common internet protocols.

### Security

Green PHY offers full SE 2.0 security interoperability modes with ZigBee SEP 2.0.  Green PHY will use a centralized root of trust model to identify and mutually authenticate nodes joining the network.  Green PHY will use Elliptic Curve Cryptography (ECC) certificates to represent identity at nodes in the network; a NIST approved and standardized mechanism for embedded device asymmetric cryptography.  The IETF standard Extended Authentication Protocol (EAP) will be used to authenticate communications nodes joining using their certificates.  IETF standard Transport Layer Security (TLS) is used to manage secure sessions and the Advanced Encryption Standard (AES) 128-CCM mode encryption will be used to encrypt communications both across station to station links and for applications across ZigBee networks.  ZigBee security has recently been harmonized with these very typical standards to enable security interoperability with any other adjacent networks.

### Power Consumption

HomePlug Green PHY has been designed to operate at a maximum power consumption of half of a Watt for Green PHY networks.  Green PHY has extended features to preserve power consumption properties under hybrid HomePlug 1.0 and HomePlug AV, and P1901-fft networks.  Green PHY achieves most of its power consumption through MAC layer collaborative sleep mechanisms.  Green PHY has features for enabling transmit power control to help reduce powerline radiated and conducted emissions as well as reduce power consumption.

# Overall Technology Evaluation Outcome

This section compares each of the three top technologies with respect to appliance centered communications technology high level systems criteria.  Individual high-level criteria will be described and each technology’s performance will be described.

## Proven Interoperable PHY/LINK/NET

Wi-Fi, ZigBee, and HomePlug Green PHY are proven interoperable technologies based on open specifications with a high degree of standards reuse.  Wi-Fi and ZigBee are the most proven technologies of the three.  Of the three technologies, only Wi-Fi is holistically standardized under the IEEE 802.11 family of standards.  ZigBee’s underlying MAC and PHY are fully standardized under IEEE 802.15.4 and current revisions are moving to an IETF RFC based layering of standards for the network layer.  Green PHY is not yet standardized, but uses and extends a core set of features in P1901-fft for its PHY, MAC PLC layering as well as IETF RFC layering for its network layer.  All the technologies have multiple representative test labs available through their respective commercial technology alliances.  Each of the three technologies has been identified by the NIST Smart Grid Interoperability Panel (SGIP) as interoperable and suitable for use in Smart Grid applications.  Each technology is currently being actively retooled to support at least one relevant Smart Grid application protocol such as the ZigBee / HomePlug Smart Energy 2.0 application profile.

## Accepted Technology in the Marketplace

It is desirable to make appliances integrate with the technology a customer is already likely to have at a premises. Technologies currently appear in several different marketplaces relevant to smart grid.  ZigBee and HomePlug are the furthest along in being retrofitted to work with AMI meter markets.  Though several companies have announced support, only initial forays into Wi-Fi for AMI meters have been demonstrated.    Premises having the internet are currently most likely to have Wi-Fi in home internet routers.  Initial forays into home internet routers/gateways having HomePlug and ZigBee have been made, but they are rare by comparison.  Utilities have considered deploying internet connected demand response gateways to help seed the market for demand responsive energy services.  In that submarket, ZigBee and Wi-Fi are leading with a strong demonstrated use of ZigBee to speak with AMI meters and various types of controls and Wi-Fi more likely reaching out to connect ZigBee to customer owned infrastructure.  Finally we looked at what technologies the customer uses in their home. Two major applications exist, connecting consumer end electronics and connecting home automation controls and sensors.  Consumer electronics are most likely connected by Wi-Fi, though ZigBee is entering the set top box market.  HomePlug Green PHY is designed to be highly relevant to the home automation and control market, and will have the ability to interact with other HomePlug variants that are very likely to appear in the consumer electronics market.

## Acceptable Cost Trade-offs

Technology components have various cost trade-offs with respect to appliance requirements.  In general dollar cost ranged from ZigBee having the lowest cost offerings to HomePlug and then to Wi-Fi with all three meeting or approaching manufacturer requirements.  Wi-Fi solutions offered the highest bandwidth, followed by HomePlug, and then ZigBee with all meeting or exceeding the bandwidth required for appliance Smart Energy Applications.  HomePlug had the lowest potential latency, followed by Wi-Fi and then ZigBee.  All three technologies outperformed single link expectations for appliance Smart Grid application latency requirements.  HomePlug was clearly designed to mandatorily support energy proportional power consumption while ZigBee and Wi-Fi enabled it in various different forms on different market offerings.  In general, the expectation that a Wi-Fi access point (AP) would be required and responsible for being capable of managing its low power mechanisms lowered our assessment of its proportionality.

## Easy to Provision / Plug-and-Play Appliances

Appliance manufacturers require the highest ease of initial connectivity for the customer; this includes joining, security establishment, repeating and routing, bridging, and link state feedback.  ZigBee and HomePlug have been designed to support all consumer and utility install cases by having common link defaults, 3rd party security management, installer codes, and full support for application layer constructs that enable easy and secure joining in Smart Grid applications.   Wi-Fi has the ability to be deployed with many of the features above, but typically requires an involved user configuration process requiring host computers, and is even more involved to establish utility grade security.  It is expected that this situation will change as Wi-Fi grows into a common Smart Grid role, and application and interaction models are adopted.  All three technologies provide standard options for signal repeating and individual link extension to support extending appliance connectivity.  Wi-Fi has a repeating mode.  HomePlug has mesh-under repeating and routing.  ZigBee has a repeating mode.  All three technologies offer standard means for bridging onto other media that may be installed in the customer’s home. This enables appliance customers to penetrate walls with one technology and propagate to areas without wires with another, and connect to existing infrastructure with any possible should such be required.  ZigBee will have route-over 6LoWPAN gateway support.  Wi-Fi is 802.2 capable, bridging trivially to other Ethernet-like media, and supports WDS wireless bridging inherently.  HomePlug is 802.2 capable, bridging trivially to other Ethernet-like media.  Finally, all three technologies have sufficient provisioning feedback support enabled in the technology to enable high detail provisioning error recovery information to customers trying to understand what steps need to be taken to connect their appliances to the Smart Grid.

## Minimizes Enabling Infrastructure

When appliance customers install the appliance, there should be little additional hardware and materials required to get started.

To minimize overhead cost to the customer due to additional stations and simplify installation and maintenance, it was preferred that individual stations provide the highest home coverage possible.  Wi-Fi and HomePlug did the best in this category as they have been designed to have higher end signal processing, more aggressive coding, and spreading mechanisms for wider passbands.  ZigBee’s mesh architecture has resulted in a radio designed for denser station count deployments to achieve high reliability lower power consumption, and reduced interference at a lower cost.  While ZigBee link budgets sometimes exceed Wi-Fi link budgets, the intent of that technology is to operate as a low power mesh.  A single HomePlug station generally covers 200 ft. out both legs of 240 / 120 V wiring schemes without issues up to and including 24 V HVAC segments.

To minimize cost and complexity at installation, technologies that required little additional cabling were preferred.  In general, Wi-Fi, ZigBee, and HomePlug all require no additional wiring to deploy.

Installation topology was considered in how it minimizes additional stations in the network.  While certain modes of Wi-Fi are considered to have high single station home coverage, its star type topology results in expensive interference and mandatory central nodes at points of branching.  This central access point in Wi-Fi was considered to create issues for where such a device would occur in a customer’s premises.  Would the utility, appliance manufacturer, or customer be required to deploy it to get the appliances to work with Smart Grid?  In contrast, HomePlug and ZigBee nodes have the capability to deploy in a mesh topology and could be deployed as ‘working’ nodes with branch points only required for devices serving as bridge points, removing the requirement for defacto customer to provide an intermediate device to get started.

Finally, appliance communications should require very low amounts of power to help enable other features on the device in various regulated low power modes.  EPA Energy Star certifications and other regulatory markets cause appliance manufacturers to need the lowest possible power consumption in the feature classes considered.  In general, ZigBee was the lowest potential power consumer that met other requirements at tension with power such as cost, data rate, and security.   Low-Power Embedded Wi-Fi was the second lowest potential power consumer and HomePlug was third.  All three technologies had peak power consumptions at or below half a Watt.

## Technology Supply Chain Exists

Each technology was evaluated for supply chain maturity.  In general, ZigBee, HomePlug, and Wi-Fi have well established integration talent, integration tools, and the parts are available for integration into Smart Grid applications.  More importantly, each has multiple independent silicon and software stack providers with a projected volume availability of silicon and stacks within an immediate horizon.

## Future Proof

Appliances need to expect that current and future revisions of communications technologies they implement will have longevity in the market, but that if they do not, will have generational compliance features and firmware upgrades to extend the useful window as far as possible. Wi-Fi is perhaps the most future-proof, having demonstrated a long history of backwards compatibility and continued support of an 802.2 bridging point.  HomePlug Green PHY has been designed to be fully interoperable at low data rate with other powerline standards such as HomePlug AV and future AV revisions as well as offering an 802.2 compliant bridging point.  ZigBee has retained interoperability at the MAC/PHY layers, but has added sufficient architecture to assure future interoperability for the near future through inclusion of IPv6 support and a strong internal standards basis.

## Handles Interference and Noise

Appliances require communications technologies which handle interference and noise so that they have features that enable them to scale to multi-dwelling unit environments, coexist with other similar communications technologies, be resilient to direct interference, and attempt to reduce their own emissions where possible.  ZigBee scales the best to multi-dwelling environments currently, offering the greatest scalability for band consumed, deployments of size 500 nodes per PAN are easily achievable and installations of up to 8000 nodes in an MDU have been reported. HomePlug came in second and was designed to scale very well across shared transformer to home and endpoint densities of 128 homes per transformers and 32 nodes per home.  Wi-Fi came in third. Wi-Fi has overlapping bands and little conflict resolution for interference, in general falling back to lower coding rates very rapidly for reasonable duty cycle consumed even when there was unused portions of the spectrum.  ZigBee had no coexistence mechanisms for wireless coexistence other than some incidental avoidance strategies and conflicts directly with Wi-Fi for many cases.  Wi-Fi coexists with certain other standards such as WiMAX and Bluetooth, but it has no standard mechanism for new technologies to integrate to assure coexistence.  HomePlug did the best with its use of IEEE P1901-ISP as its coexistence mechanism.  All three technologies have means for transmit power control.  HomePlug and Wi-Fi have explicit coordinated mechanisms for transmit power control and ZigBee implementers could easily implement a mechanism for a given product using the features typically available. Finally, Wi-Fi, HomePlug, and ZigBee all had sufficient spectrum spreading mechanisms to reject narrowband interference sources.

## Easy System Operation

Appliances shouldn’t become challenging to use due to communications gear.  Communications technologies should offer stable and reliable links, plug and play network interaction, and sufficient link state feedback to the customer.    ZigBee, HomePlug, and Wi-Fi all offered stable and reliable links. For most modulation means available, the receiver error margin is relatively narrow resulting in links that work well when the link strength is high enough and do not typically behave marginally when they are approaching the edge of their range, eliminating much uncertainty about good coverage.  ZigBee, HomePlug and Wi-Fi all have demonstrated potential in the market for plug-and-play operation.   Finally, all three technologies have sufficient data enabled in the technology to enable high detail link state feedback to customers trying to understand what steps need to be taken to connect their appliances to the Smart Grid.

## Easy to Maintain

Appliance communications technologies should be easy to maintain.  Most consumer technologies support this goal, aiming to be maintenance free once installed beyond adding additional customer devices to the network. ZigBee, HomePlug, and Wi-Fi all do very well, and are very low maintenance once installed.

## Easy to Replace

Appliance communications technologies should be easy to replace.  Beyond installing and registering new authentication credentials following a replacement, there is nothing specific that would prevent ZigBee, HomePlug, and Wi-Fi from being easy to physically replace.  Installation issues are covered in other criteria.  Therefore, this remains a manufacturer issue to differentiate.

## Robust Messaging Capability

Appliance communications should be robust to changing home area network conditions. To address certain smart grid applications, technology features are required to honor quality of service (QoS) and use effective error correcting codes to address changing network conditions.  In general, quality of service may be negotiated or designed end-to-end, but must be enforced at individual stations on a network.  Wi-Fi and Green PHY both have means to enable quality of service through support of IEEE 802.1Q delivery at the link layer.  ZigBee had no such QoS means.  All three mechanisms offer sufficient means for error correcting codes.  Wi-Fi and Green PHY utilized more aggressive coding and forward error correction codes due to the higher data rates they achieve.

## Affordable to Install

Appliances need technology that is more affordable for the customer to install.  ZigBee and Green PHY are peer-to-peer technologies, and therefore only require additional stations if there are link reachability issues. Wi-Fi is a star topology technology and requires that the customer or other entity provide an access point to connect individual appliances.  Therefore, cost to install is very likely higher for the average case.

## Sufficient Enabling Security Mechanisms

Appliances need security both to allow the customers to protect themselves and to attach appliances directly to utility and third party networks that may have minimum-security expectations.  In general, the market expects that network stations be capable of authenticating themselves using pre-shared keys and certificates, offering link and network encryption options, and enforcing logical network separation.  Wi-Fi, ZigBee and HomePlug offer means to enable best of breed EAP-TLS based mutual authentication and encryption methods.  Each technology has customer-centered features for establishing pre-shared-keys and centralized means for establishing public key infrastructure X.509 certificates.  Further, encryption links supporting AES-128-CCM are possible for links, networks, and applications.   Finally, all three technologies support one or more forms of logical network separation.  Both Wi-Fi and Green PHY support 802.1Q style vlans while Zigbee uses 802.15.4 PANs for network separation.  All three enable network separation enforcement through encryption.

# Conclusions

In the technology survey, we looked very closely at many technologies including the three featured here.  We considered their individual merits as well as overall smart grid relevance.  Appliance manufacturer criteria and customer appliance criteria determined that low power, low cost, easy to use, security conscious and Smart Grid relevant technologies dominate other technologies.  Appliance manufacturers are not alone in their considerations of manufacturability and the customer condition. In general, technologies specifically adapted to support Smart Grid applications at the electric meter and in home area networks outperformed those that are not.  Smart Energy 2.0 bearer class technologies such as ZigBee, low power embedded Wi-Fi, and HomePlug Green PHY present the market with viable full-spectrum solutions for smart grid integration into appliances as well as support for longevity considerations both in smart grid and appliance markets

.